Zmodem Rev 16.01

Copyright (c) 1989, 90, 91, 92, 93

By

International TeleCommunications

ALL RIGHTS RESERVED

Table Of Contents

i

Limited Warranty .......................................... Page   1

Distribution Policy And Copyright ......................... Page   1

Copyright .............................................. Page   1

-- Test-Drive Distribution Policy -- ................... Page   2

Introduction .............................................. Page   2

Zmodem Command Line Switches .............................. Page   3

/S ..................................................... Page   3

-T<d,w,o> .............................................. Page   4

/R ..................................................... Page   6

/U ..................................................... Page   6

/F ..................................................... Page   6

@ ...................................................... Page   7

/K ..................................................... Page   7

/N ..................................................... Page   8

/P ..................................................... Page   8

/V ..................................................... Page   8

/B ..................................................... Page   8

/L ..................................................... Page   9

Environment Lock Variable ........................... Page   9

Required Switches ......................................... Page   9

Your Receive Batch File ................................ Page  10

Send Batch File ........................................ Page  10

Error Levels .............................................. Page  10

Bbs/Terminal Programs ..................................... Page  10

Serial Drivers And Multi-Port Cards ....................... Page  11

Intelligent Multi-Port Stargate ........................ Page  11

Intelligent Multi-Port Arnet ........................... Page  12

Intelligent Multi-Port Digiboard ....................... Page  12

Fossil ................................................. Page  13

Non-Intelligent Serial Cards ........................... Page  13

Non-Intelligent Digiboards ............................. Page  13

Setting Up  The Digi Pc/4 Board ..................... Page  14

Setting The Jumpers .............................. Page  14

Multi-Board Connectors ........................... Page  15

Dip Switches ..................................... Page  15

Setting Up A Digiboard Pc/8 ......................... Page  16

Setting The Jumpers .............................. Page  16

Multi-Board Connectors ........................... Page  17

Dip Switches ..................................... Page  17

Setting Up The Digiboard Pc/16 ...................... Page  18

Non-Intelligent Stargate ............................... Page  20

Non-Intelligent Ast .................................... Page  20

Non-Intelligent Boca ................................... Page  21

8259 Interrupt Management ................................. Page  21

Zmodem License ............................................ Page  22

Zmodem          Technical Reference Manual          Page   1

Limited Warranty

This   products provided "as is" without  warranty   of   any

kind.   The entire risk as to the results and performance  of

the  program  is assumed by you.  Should  the  program  prove

defective,  you  (and not International TeleCommunications  )

assume the entire cost of all necessary  servicing, repair or

correction.   Further, International TeleCommunications  does

not warrant, guarantee, or make representations regarding the

use of, or the results of the use of this program in terms of

correctness,    accuracy,   reliability,   currentness,    or

otherwise;  and  you  rely on the program  and  its   results

solely   at  your own risk.  International TeleCommunications

cannot  accept   responsibility for system  damage,  loss  of

profit,   or   any    other    special,    incidental,     or

consequential damage  resulting  from  the use or   inability

to  use  this product.

International  TeleCommunications  DOES   warrant   to    the

original  licensee of a REGISTERED product that  the  program

disk(s)  on which  the  program  is  recorded be  free   from

defects   in materials and workmanship under normal  use  and

service  for  a period  of  ninety  (90) days from  the  date

of   delivery   as  evidenced  by  a copy  of  your  receipt.

International  TeleCommunications' entire liability and  your

exclusive  remedy  shall  be replacement  of  the   disk  not

meeting  International TeleCommunications Limited Warranty.

Distribution Policy And Copyright

Copyright

Despite  the method of marketing, Zmodem is not in   the

public  domain.    It  is the  proprietary  property  of

International  TeleCommunications.    All   rights   are

reserved.      Copying,  duplicating,     selling     or

otherwise    distributing    this product    except   as

expressly   permitted  in the   TEST-DRIVE  Distribution

Policy  below   is a  violation  of  the  Law.  However,

we grant you  the right,  in fact encourage you  to make

and  distribute as  many copies   of   this   TEST-DRIVE

version   as   you    wish,   using    any    acceptable

medium  of exchange, with the following provisions:

Zmodem          Technical Reference Manual          Page   2

-- Test-Drive Distribution Policy --

1.   Please feel free to distribute this TEST-DRIVE version as

often as you like, to any interested parties.

2.   Please do not distribute the program without all of its

original related files, addendum files, and documentation.

3.   Please obtain our written permission prior to distribution

with any "Bundled" software packages.

4.   Please do not accept payment for the program; a copying fee of

$10.00 per diskette may be charged when distributed by a

non-profit user-group.

5.   Please do not alter the program or documentation in any

manner.

6.   NOTE - Individuals, groups, vendors, or other software

distribution companies which market diskettes containing

the TEST-DRIVE programs for profit must obtain written

permission from International TeleCommunications prior to

distribution.

Introduction

Zmodem.exe  is  a  full  function  stand  alone   Zmodem

protocol driver.

Zmodem.exe offers exceptional fast Zmodem file transfers

and   it  fully  supports  ZSKIP,   Resume   interrupted

Transfers,  32  and  16 bit CRC  error  detection,  8259

interrupt  management, and a wide range  of  intelligent

and  non-intelligent serial cards (also  support  Fossil

and has an internal driver).

Zmodem.exe  is  one  of the fastest  (we've  been  told)

Zmodem drivers you are likely to find.

It  is  the only stand alone zmodem driver  that  offers

support for such a WIDE range of serial card as well  as

8259 interrupt managment (faster transfers).

Zmodem          Technical Reference Manual          Page   3

Zmodem Command Line Switches

Each  switch must be preceded by either a forward slash   '/'

or a dash '-'.

O   Should Always Be Used With Osiris BBS.

V   Engage ANTI-PMS Screen, For Use With Osiris BBS Only.

S   Use RTS flow control.

K   Keep Aborted Transfers.

@   Use Send Control File.

U   Place Uploads In This Directory.

R   Receive Files

N   Osiris Node Number, Osiris BBS Only.

L   Lock Serial Rate.

B   Connect Baud Rate.

P   Serial Port.

F   Send These Files.

T   Time Slicing.

/S

9600 bps and higher

Slow Computer

If   you have a high speed modem you MUST have a   16550

Uart. Attempting  to run at high speeds without one will

result  in errors.

You  might sneak by without a 16550 if you have a   fast

386, or 486.

A  good way to check is by watching your  transfers.  If

you  are under  an  error correcting connect,  then  you

should   almost  NEVER  get an  error  reported  by  ANY

transfer protocol.

Getting   errors  reported (by ANY  transfer   protocol)

while under  MNP  or  ARQ  can only mean.

1.  The protocol goofed (Zmodem doesn't goof).

2.  You   may  have another   application   interfering

with serial interrupts.

3.  You have hardware problems.

4.  You do not have a 16550 Uart.

If   you have a 16550 installed, then you can  rule  out

number four  and that leaves you with two or three.

If you have a multi-tasker or a network and a 16550 then

the network and/or multi-tasker is stealing to much time

and  the 16550 fifo's are being over run.

Zmodem          Technical Reference Manual          Page   4

If you have a 16550 and NOTHING else that may  interfere

with  the   serial  interrupts,  then  you   more   than

likely  have hardware problems.

If  it is option 4, buy one and install it. 16550  uarts

cost about $20.00 and are WELL worth it.

Unless   you   have  hardware problems, I can  HELP  you

by  kicking   in RTS flow control. This does  slow  down

transfers,  but  it  is better to slow  them  DOWN  than

experience errors.

If   you have to, add /S to the command line.  But  only

use   it if  you  HAVE  to and only until you  fix   the

CAUSE  of  the problem.

-T<d,w,o>

This  governs  the  TYPE of time  slicing  the  protocol

driver will use. Leaving off the -T switch means no time

slicing.

D= Desqview or Topview

W= Windows

O= OS-2

E.g. -TD, -TW, -TO

While time slicing does reduce the drain on your CPU  it

may,  depending  on  the  multitasker  (OS-2,   Windows,

Topview, or Desqview) and your hardware, greatly  reduce

transfer rates.

Despite   the  term  "Multitasking",   multitasking   is

impossible using a single CPU. The CPU can only  process

a single instruction (2 integer instructions if the  new

Intel  P5  chip is used) at once.  Multitasking  with  a

single CPU is nothing more than quickly swapping in  and

out  programs giving each a short period of time to  run

before swapping in another one.

For  instances,  when  Windows  swaps  away  it  freezes

everything in that windows and this includes ANY  device

drivers  or interrupt service routines. If you  use  the

internal serial driver, it to is frozen and cannot  send

data that may be in the transmit buffer.

Zmodem          Technical Reference Manual          Page   5

While all multitaskers do this, some are more  efficient

than  others. Some will give interrupt  services  higher

priority  than  other tasks and allow them to  use  much

more CPU time. Even if the application only gets  access

to  the CPU once ever oh . . . 9 clock  tics,  interrupt

routines used by that application may get access to  the

CPU every 2 clock tics. This is sometimes referred to as

multi-threading.

There  are  conditions when it doesn't  matter  how  the

multitasker  works.  For instances,  intelligent  serial

cards.

If  you  have  an intelligent serial  card,  it  handles

sending  the data and the multitasker does not  stop  it

from doing so by swapping out the window. In this  case,

always use Time slicing.

Fossil drivers can sometimes get around this if they are

loaded    BEFORE   the   multitasker.   However,    some

multitaskers will not allow dual access, from  different

windows, to the fossil.

To determine if your system works better with or without

time slicing, testing is the only way.

Do uploads and downloads using time slicing and  without

using  time slicing. If you run a single  node,  compare

the transfer rates.

If you run multiple nodes, it gets more involved.  After

getting  the  first set of figures,  repeat  the  entire

process but make sure you have someone doing the uploads

and downloads on ALL nodes at once. THEN compare ALL the

transfer  rates. Doing multiple transfers at  once  DOES

effect  the  outcome. You may find that  while  transfer

rates drop drastically with a single transfer, they  may

actually go up when you have multiple transfers going at

once.

When you are doing the tests, keep in mind that transfer

rates WILL vary, but only a little bit. Look for the big

differences and don't worry about 1 to 3 percent.

Zmodem          Technical Reference Manual          Page   6

/R

Receive Files

When   I  encounter  this switch, I  will  go   directly

to receive files mode.

You   MUST  make sure that /R is the  very  last  switch

used.  If it  is not, then you will be missing important

command  line switches.

/U

Place inbound files in this directory

You   can  direct Zmodem to place inbound  files  in   a

specific directory by using the /U command.

I will verify the directory you specified, so make  sure

it is a VALID directory.

You   cannot use Drive  designators or ROOT  directories

with the /U switch.

For   instance,    /UC:  or  /UC:\. C:  and  C:\  aren't

listed in the FAT and I cannot validate them.

Make   sure  you do NOT use a trailing   backslash  with

the   /U path, I'll ADD a backslash when I concate   the

file name with the path you specified.

E.g., d:\uploads\ is NOT valid, it should be  d:\uploads

Here    is   a   sample   of   a   GOOD   /U    setting:

/UC:\TELIX\UPLOADS

/F

Send These Files

When   I   find  the  /F,  I  will  immediately    begin

sending.

YOU   should  make  sure that you have listed  ALL   the

OTHER commands  before using /F. Make sure you place ONE

blank  space  between  the  /F  switch  and  the   first

filename.

Place one blank space between ADDITIONAL files names, if

you  have  specified more than one. You  may  include  a

drive and path with the file names.

Zmodem          Technical Reference Manual          Page   7

E.g.,  c:\uploads\file1.exe is a valid file name.

You can use the DOS wild card characters '?' and '*'.  I

will  expand  those  characters to match  any  file   in

that directory.

I   do   not place limits on the number of files    that

can  FOLLOW  /F,  but  DOS DOES! DOS    restricts    the

number  of characters you can place  on  a command  line

to 128.

DOS   also limits you to nine batch parameter   symbols,

%1 through %9.

@

I'll assume USE this file as  a  "SEND"  control file.

e.g. -@D:\OSIRIS\SEND2.CTL

Since   DOS forces me to work with 128 and   nine  batch

variables,  using  @  to  send   file   names   is   the

preferred method of passing the names.

The  information following  @ is the full  drive,  path,

and file name of a TEXT file.

I   do  not place restrictions on the  number  of  files

that you  can  have listed and you can include the  full

drive,   path,  along  with name of the  file.  You  can

include DOS wild card characters, '?' and '*'.

E.g., C:\PCboard\Tlx310a.exe is valid.

E.g., C:\Telix\*.arj is valid.

Each line in the file must contain only ONE filename and

has to end with the DOS CR/LF sequence.

Sample Send Control File:

C:\DRIVERS\MPT110.ZIP

C:\DRIVERS\TMOD610.ARJ

C:\DRIVERS\ZMOD*.*

/K

Keep Aborted Transfers

If   you  drop  carrier or time out, I will  delete  the

aborted file.

Zmodem          Technical Reference Manual          Page   8

If  you're  using Zmodem with a TERMINAL program,   then

you  will  WANT to keep the file. Adding   /K   to  your

Zmodem command line tells  ME  you  want to keep aborted

transfers.

/N

Node Number

If    you're  running OSIRIS, you can include   /Nx   on

the command line.

E.g., /N1  /N2  /N3  . . .  /N255

Add /N*N to your protocol command lines.

I'll    display the callers name, city, and  state    at

the bottom of the screen, useful when you walk into  the

room  and WONDER who  is currently downloading what.

/P

Com Port

You   have to tell me which serial port your  caller  is

ON  by including /Px on the command line.

E.g., /P1  /P2  /P3  . . .  /P8

/V

Engage PMS Screening

This   can ONLY be used on an OSIRIS system. This  tells

me   to allow  Osiris  to monitor the serial  port  with

its   ANTI-PMS  screening  system.

/B

Connect Baud Rate

You have to provide the TRUE CONNECT RATE, most  of  the

time this is the same as your SERIAL baud rate.

e.g. /B1200  /B2400  /B9600

If you have a high speed modem with a FIXED serial rate,

the CONNECT  rate may not always be the same, but   that

doesn't  change  anything. You still  should  pass   the

TRUE connect  rate and not the SERIAL rate.

Zmodem          Technical Reference Manual          Page   9

/L

Lock Rate

This tells me you have a LOCKED Serial Port.

E.g.

/L38400

/L57600

/L19200

Environment Lock Variable

You can define an  environment    variable   called

COMx=BaudRate.  Replace  x  with the   serial  port

number   and BaudRate with the  LOCKED  baud rate.

e.g. COM1=19200

Do  NOT add blank spaces or comma's. Make sure  you

use  Upper Case.

You can do that in the AUTOEXEC.BAT file or in  the

BATCH file you use to CALL Zmodem.

e.g.

cls

SET COM2=38400

Zmodem -B%1 -P%2 -R

Exit

Required Switches

Each  Zmodem installation will require TWO switches. It  does

not matter  what type of BBS/Terminal program you're going to

use.

It does not matter if it is SEND or RECEIVE.

/Bxxxxx and /Px

You  should  replace   xxxxx  in  the /B  with   the  correct

CONNECT rate or 0. Replace the x in /P with  the correct  COM

port.

/B9600 /P2  (Connect Rate 9600, COM 2)

/B2400 /P1  (Connect Rate 2400, COM 1)

It is not important WHERE you place the switches, as long  as

they are NOT the LAST switch used.

Zmodem          Technical Reference Manual          Page  10

Your Receive Batch File

Your   Receive   Batch  file  will  require   only   one

additional switch, /R.

It MUST be the very last switch on the command line.

The  MINIMUM command line for receiving would be:

Zmodem /B2400 /P1 /R

2400  and  1 can  be supplied by DOS  batch   variables,

e.g. Zmodem /B%1 /P%2 /R

Other   optional   switches,  /U  . . .  /K  etc.,  must

be included BEFORE the /R switch.

Send Batch File

Your  "SEND" batch file only "requires"  ONE  additional

switch, /F or -@.

The MINIMUM command line for sending a file:

/B2400 /P2 /F FILENAME#1 FILENAME#2 FILENAME#3

or

/B2400 /P2 -@CTRL_NAME

2400,   2,  FILENAME#1, FILENAME#2,   FILENAME#3,   etc.

may  be supplied by  DOS  batch variables.

E.g., Zmodem /B%1 /P%2 /F %3 %4 %5 %6 %7 %8 %9

Error Levels

I  will  exit,  following the transfer, with  an error  level

of 0  or  1.

Zero,  0,  indicates  a  successful  transfer  and  one,   1,

indicates an error.

Bbs/Terminal Programs

I  use  a very basic command line setup  which  should  allow

Zmodem to be used by almost any program. I do NOT expect that

it will work with EVERYTHING.

If  you are using something that it will just flat  not  work

with then send it to me. You  can upload it or mail it and  I

will attempt to find  out why.

Zmodem          Technical Reference Manual          Page  11

Serial Drivers And Multi-Port Cards

Zmodem  supports  Intelligent multi-port serial  cards,  non-

intelligent  multi-port  serial cards, fossils,  and  has  an

internal  serial driver (fastest method of dealing  with  the

serial port).

Zmodem  defaults to the  internal high speed driver, but  you

can  change the driver type by using the use the  environment

variable COMDRV, ONLY AVAILABLE IN REGISTERED VERSION.

COMDRV=FOSSIL

{Intelligent Serial Cards}

COMDRV=STARGATE

COMDRV=DIGI

COMDRV=ARNET

{Non-Intelligent Serial Cards}

COMDRV=NDIGI

COMDRV=NSTARGATE

COMDRV=AST

COMDRV=BOCA

Intelligent Multi-Port Stargate

Load  the  Star  Gate device driver,  ACL.SYS,  in  your

config.sys.  When  your  system  boots,  run  ACLDL   to

download your code.

Zmodem  defaults to the 16 port with the I/O address  of

0x200  at D000:000 starting at COM 5. If  your  hardware

doesn't  match this, you'll need to create a  SERIAL.CFG

control file in SAME directory that houses Zmodem.Exe or

re-configure your stargate board.

SERIAL.CFG for STARGATE Intelligent Multi-port Cards:

The  first line of this file should contain 5  settings,

separated by a single space.

BOARD BOARD_SEGMENT PORT FIRST_PORT_NUMBER NUMBER_OF_PORTS

Board:  Stargate  board to use (0 - 3).  This  tells  us

which board you want to use.

Board_segment:  Segment of the Star Gate board's  memory

window.

Zmodem          Technical Reference Manual          Page  12

Port: The Address of the board's I/O latch.

First_port_number:  Name given to the first port on  the

board  (1,2,3,etc). The designation of the  port  number

for the first port on the board is arbitrary.

Number_of_ports: Number of ports on the board.

Example:

{SERIAL.CFG}

0 0xd000 0x300 4 8

Intelligent Multi-Port Arnet

When  you  load  the Arnet device  driver,  port  number

assignments  are printed on the display. These  are  the

number you'd use for the zmodem serial port parameter.

Use  the /N5 switch on ASP.SYS. If you do not  use  this

switch,  attempts to set handshaking will result  in  an

error.

The  files, box.bin and fastbios.bin, supplied with  the

Arnet boards, must be in your root directory for this to

work  properly. These files are used by ASP.SYS at  boot

time.

Intelligent Multi-Port Digiboard

Run  the Digiboard config. program (XIDOSCFG.EXE)  which

asks you for the desired port numbers. Also, install the

device  driver  supplied with the board. You  must  also

complete the following steps:

1. Set up the board NOT to use interrupts.

2. Setup to support INT 0x14 for most drivers and  EBIOS

for the XIDOS5 driver.

3.  Change  the configuration to enable ports,  if  your

driver  requires  it. Make sure "NONE" is  the  selected

handshaking for ports.

4.  Reboot  your  system. During the  boot,  the  driver

should   respond   with   the   message    "successfully

initialized board".

Zmodem          Technical Reference Manual          Page  13

Fossil

Any  fossil  driver will work.  See  documentation  that

comes with the fossil driver for details.

Non-Intelligent Serial Cards

Non-Intelligent  boards  frequently  come  with   device

driver  software. This software provides INT 14H  access

to  the  UART on the board.

Do N O T use this software with Zmodem. Zmodem  accesses

the  hardware directly without the use of  any  external

drivers.

Non-Intelligent Digiboards

(COMDRV=NDIGI)

The  Digiboard  DigiCHANNEL PC/16, PC/8,  and  PC/4  are

multi-channel  communications  boards  that  provide   a

serial  communications  subset for  IBM  and  compatible

computers.  Each  of  the  boards  supports  full  modem

control  on  each port. Up to two DigiCHANNEL  PC/16,  4

PC/4  or  PC/8  boards  can be  installed  on  a  single

computer providing up to 32 additional ports.

To   insure  connection  flexibility,  3   options   are

available:

1. An I/O mate connection box with external connections.

2. An octopus cable with four or eight DB25 connectors.

3.  An  RJ45  connection box with  eight  RJ45  or  RJ11

connectors.

The   DIGI  PC/4  and  PC/8  addressing  and   interrupt

selection  are flexibly and not limited to preset  IRQs,

address,  or  address  ranges.  The  user  may  set  DIP

switches to select different status registers (DS1)  and

I/O  port  addresses  (DS2 - DS9), as  well  as  setting

jumpers to select either one or two interrupts.

The DIGI PC/16 offers hardware jumpers for selecting  an

interrupt  and  a PAL with preset I/O  port  and  status

register addresses.

There  are  six possible system interrupts that  may  be

selected for the DIGI PC/X board: IRQ2 - IRQ7.  However,

IRQ3  and IRQ4 are commonly reserved for  standard  COM1

and COM2 serial hardware.

Zmodem          Technical Reference Manual          Page  14

If  multiple  boards are installed in one  system,  each

board is given a unique I.D. such as board 0 and board 1

by setting the board ID jumpers. The status register and

IRQ  of  ALL boards can be set to the same  values,  and

the  boards are daisy changed with special  cables  that

pass  signals  from the interrupt sequence.  When  daisy

chained,  the  boards are treated like  one  board.  The

boards  can  also be setup  independently  on  different

interrupts.

Setting Up  The Digi Pc/4 Board

The  COM/4  board  has 4  serial  interface  ports,

individual address and interrupt settings, and  two

8 bit status ports. It is preset and tested at  the

factory  at IRQ3; address 100, 108, 110,  and  118;

status port ON, and set to address 140. Check  your

system for any peripherals that may use either  the

preset  interrupt  or  addresses. If  there  is  no

conflict,  you can install the COM/4 board  without

modifications.  However,  if  you  need  to  change

settings, details are provided below.

Setting The Jumpers

Jumpers  1-4 are used to choose the  interrupt

for each individual port.

For odd interrupts IRQ3, IRQ5, or IRQ7 set the

shunt   between   pins  1  and   2   for   the

corresponding jumper.

For  even interrupts IRQ2, IRQ4, or  IRQ6  set

the  shunt  between  pins  2  and  3  for  the

corresponding  jumper. Ports 1 through  4  map

with jumpers 1-4 respectively.

J9  and  J10  are used to  set  the  board  ID

number.  The  setting will depend  upon  which

board  it represents. If more than  one  board

is used in the system, each board must be  set

to  a unique identification number. The  first

board  should always be set to 0. If you  have

daisy  changed more than one  board, you  will

use  the  board #s 1-3. The  settings  are  as

follows:

Zmodem          Technical Reference Manual          Page  15

Board #            J9                       J10

------------------------------------------------------------

0        between pins 2 and 3     between pins 1 and 2

1        between pins 2 and 3     between pins 1 and 2

2        between pins 1 and 2     between pins 2 and 3

3        between pins 1 and 2     between pins 1 and 2

J85  through  J90  are used  to  select  which

interrupt  the  board  will  use.  Set   these

jumpers as follows:

IRQ#               Jumper#

---------------------------

2                  J90

3                  J85

4                  J89

5                  J86

6                  J88

7                  J87

Multi-Board Connectors

P2 and P3 are jumpers used to daisy chain  two

boards  together  using  a  special  DigiBoard

cable. If you are only using one board,  place

a  shunt  on  pins 2 and 3 of  both  of  these

jumpers. This is VERY important. Without  this

shunt, no interrupts will be received.

Dip Switches

Address line A3 through A9 determine the  port

address  and  are logic-selected  on  the  DIP

switches  for  each  port  for  switches   DS3

through  DS5. The DIP switches for  each  port

can  be set to any NON-CONFLICTING address  in

the I/O map from 0000H to 03FFH.

Switch positions 1-7 switch set the address of

each port. Position 1 corresponds to A9, 2=A8,

3=A7,  4=A6, 5=A5, 6=A4, 7=A3. Position  8  of

each  switch enables or disables the port.  We

recommend  assigning consecutive port  numbers

for easy of use.

The  following table lists  suggested  address

and   switch   settings  for   the   DigiBoard

DigiCHANNEL PC/4 board:

Zmodem          Technical Reference Manual          Page  16

Port#   Base    Dip                  Switch Settings

Address Switch   1    2    3    4    5    6    7    8

----------------------------------------------------------------

Status 140H    DS1      ON   OFF  ON   OFF  ON   ON   ON   ON

port

----------------------------------------------------------------

1     100H    DS2      ON   OFF  ON   ON   ON   ON   ON   ON

2     108H    DS3      ON   OFF  ON   ON   ON   ON   OFF  ON

3     110H    DS4      ON   OFF  ON   ON   ON   OFF  ON   ON

4     118H    DS5      ON   OFF  ON   ON   ON   OFF  OFF  ON

Position   (Switch  setting  8)   enables   or

disables the port where ON = enabled.

Setting Up A Digiboard Pc/8

The   PC/8  board  has  eight   individual   ports,

individual address and interrupt settings and two 8

bit status ports.

The PC/8 board is preset and tested at the  factory

using  IRQ3; address 100, 108,110, 118,  120,  128,

130,  and  138; status port ON and set  to  address

140.

Setting The Jumpers

Jumpers J1-J8 are used to choose the interrupt

for  each  port.  For  odd  interrupt  numbers

including  IRQ3, IRQ5, or IRQ7, set the  shunt

between  pin  1 and 2  for  the  corresponding

jumper.

For  even  interrupts, set the  shunt  between

pins  2 and 3. All ports HAVE to use the  same

interrupt to work properly. Ports 1 through  8

map to jumpers 1-8 respectively.

J9  and  J10  are used to  set  the  board  ID

number.  The  setting will depend  upon  which

board  it represents. If more than  one  board

is used in the system, each board must be  set

to  a unique identification number. The  first

board  should always be set to 0. If you  have

daisy  changed more than one  board, you  will

use  the  board #s 1-3. The  settings  are  as

follows:

Zmodem          Technical Reference Manual          Page  17

Board #            J9                       J10

------------------------------------------------------------

0        between pins 2 and 3     between pins 1 and 2

1        between pins 2 and 3     between pins 1 and 2

2        between pins 1 and 2     between pins 2 and 3

3        between pins 1 and 2     between pins 1 and 2

J85  through  J90  are used  to  select  which

interrupt  the  board  will  use.  Set   these

jumpers as follows:

IRQ#               Jumper#

---------------------------

2                  J90

3                  J85

4                  J89

5                  J86

6                  J88

7                  J87

Multi-Board Connectors

P2 and P3 are jumpers used to daisy chain  two

boards  together  using  a  special  DigiBoard

cable. If you are only using one board,  place

a  shunt  on  pins 2 and 3 of  both  of  these

jumpers. This is VERY important. Without  this

shunt, no interrupts will be received.

Dip Switches

Address line A3 through A9 determine the  port

address  and  are logic-selected  on  the  DIP

switches  for  each  port  for  switches   DS3

through  DS5. The DIP switches for  each  port

can  be set to any NON-CONFLICTING address  in

the I/O map from 0000H to 03FFH.

Switch positions 1-7 switch set the address of

each port. Position 1 corresponds to A9, 2=A8,

3=A7,  4=A6, 5=A5, 6=A4, 7=A3. Position  8  of

each  switch enables or disables the port.  We

recommend  assigning consecutive port  numbers

for easy of use.

The  following table lists  suggested  address

and   switch   settings  for   the   DigiBoard

DigiCHANNEL PC/8 board:

Zmodem          Technical Reference Manual          Page  18

Port#   Base    Dip                  Switch Settings

Address Switch   1    2    3    4    5    6    7    8

----------------------------------------------------------------

Status 140H    DS1      ON   OFF  ON   OFF  ON   ON   ON   ON

port

----------------------------------------------------------------

1     100H    DS2      ON   OFF  ON   ON   ON   ON   ON   ON

2     108H    DS3      ON   OFF  ON   ON   ON   ON   OFF  ON

3     110H    DS4      ON   OFF  ON   ON   ON   OFF  ON   ON

4     118H    DS5      ON   OFF  ON   ON   ON   OFF  OFF  ON

5     120H    DS6      ON   OFF  ON   ON   OFF  ON   ON   ON

6     128H    DS7      ON   OFF  ON   ON   OFF  ON   OFF  ON

7     130H    DS8      ON   OFF  ON   ON   OFF  OFF  ON   ON

8     138H    DS9      ON   OFF  ON   ON   OFF  OFF  OFF  ON

Position   (Switch  setting  8)   enables   or

disables the port where ON = enabled.

Setting Up The Digiboard Pc/16

This  board  is, in most respects,  the  functional

equivalent of two DigiChannel PC/8 boards operating

together.  The  PC/16 features the same  method  of

interrupt  sequencing in hardware, sparing the  CPU

from having to POLL the individual ports. While the

PC/8  has several DIP switches and jumpers to  set,

the PC/16 has all addresses set in the hardware.

Default values are set at IRQ3, UART base address =

100h, and status register = 140H.

To install the board, set the jumpers including the

board  selection  jumper  and  the  IRQ   selection

jumper.

Check other peripherals for conflicts with the pre-

set addresses. If a conflict occurs, you will  HAVE

to change the I/O port on the OTHER device.

Connector  Jumper  P3 is used to  daisy  chain  two

boards together using a special Digiboard cable. If

you are only using one board, place a shunt on this

jumper.  It  is located  in  the  upper  right-hand

corner of the board.

Interrupt  Jumper J1 through J6 are used to  select

which interrupt the board will use. Set the jumpers

as follows

Zmodem          Technical Reference Manual          Page  19

IRQ#           Set the shunt at Jumper #

----------------------------------------

2                       J1

3                       J2

4                       J3

5                       J4

6                       J5

7                       J6

Set the BOARD I.D. Jumper J7 is used to number  the

board if you have more than one cascaded.

For  Board #0 set the shunt between pins 2  and  3.

For Board #1 set the shunt between pints 1 and 2.

Telling Zmodem about the non-intelligent digiboard board

requires  you  to create a SERIAL.CFG file in  the  same

directory that houses Zmodem.exe.

The  first  line  of this  file  contains  four  fields,

separated by a single space.

<IRQ> <Shared_port_status> <First_Port_Number> <Port_Count>

<IRQ> The IRQ line used by this board 3-7.

<Shared_port_status>  Address  of the  status  port  (in

HEX).

<First_Port_Number> Number of the first port. This is an

arbitrary port number.

<Port_Count> Total number of ports on the board. Be sure

to  put the correct total number of ports and  NOT  just

the  number of ports you'll be USING. E.g. 4, 8, 16,  or

32.

The  Second  line  should contain  the  PORT  ADDRESSES,

separated  by a single space, for each port on the  card

(In HEX). If you defined Port_Count as 4, then you would

have FOUR fields on this line. If you defined port count

as 16 then Sixteen fields.

Example, four port digiboard with default settings using

COM5 as the first port number.

{SERIAL.CFG}

3 0x140 5 4

0x100 0x108 0x110 0x118

Zmodem          Technical Reference Manual          Page  20

Non-Intelligent Stargate

(COMDRV=NSTARGATE)

Telling Zmodem about the non-intelligent Star Gate board

requires  you  to create a SERIAL.CFG file in  the  same

directory that houses Zmodem.exe.

Serial.cfg  contains  a  single  line  with  3   fields,

separated by a single space.

IRQ FIRST_PORT_NUMBER FIRST_PORT_ADDRESS

IRQ  The IRQ line used by this board (2-7).

FIRST_PORT_NUMBER  Number of the first port. This is  an

arbitrary port number.

FIRST_PORT_ADDRESS   Address (In HEX) of the first  UART

on the board.

Example:

{SERIAL.CFG}

3 5 0x180

Non-Intelligent Ast

(COMDRV=AST)

Telling  Zmodem  about  the  non-intelligent  AST  board

requires  you  to create a SERIAL.CFG file in  the  same

directory that houses Zmodem.exe.

Serial.cfg  contains  a  single  line  with  3   fields,

separated by a single space.

IRQ FIRST_PORT_NUMBER FIRST_PORT_ADDRESS

IRQ  The IRQ line used by this board (2-7).

FIRST_PORT_NUMBER  Number of the first port. This is  an

arbitrary port number.

FIRST_PORT_ADDRESS   Address (In HEX) of the first  UART

on the board.

Example:

{SERIAL.CFG}

3 5 0x1a0

Zmodem          Technical Reference Manual          Page  21

Non-Intelligent Boca

(COMDRV=BOCA)

Telling  Zmodem  about the  non-intelligent  BOCA  board

requires  you  to create a SERIAL.CFG file in  the  same

directory that houses Zmodem.exe.

Serial.cfg  contains  a  single  line  with  4   fields,

separated by a single space.

IRQ FIRST_PORT_NUMBER PORT_COUNT FIRST_PORT_ADDRESS

IRQ  The IRQ line used by this board (2-7).

FIRST_PORT_NUMBER  Number of the first port. This is  an

arbitrary port number.

PORT_COUNT Total number of ports on the board (4 or 8).

FIRST_PORT_ADDRESS   Address (In HEX) of the first  UART

on the board.

Example:

{SERIAL.CFG}

3 5 4 0x180

8259 Interrupt Management

The interrupt manager automatically assigns priority to  each

of  the systems interrupts. The timer counter, keyboard,  and

cascaded interrupt all have higher priority than the UART.

To  gain  the  MAXIMUM  throughput  (especially  when  multi-

tasking),  we suggest that you allow Zmodem to alter the  IRQ

manager so the UART has the highest priority.

You can do this using the environment variable 8259=IRQ{1-7}

Examples:

8259=IRQ4

8259=IRQ3

Keep  in  mind that only ONE of the uarts  (in  a  multi-uart

environment) can be switched to the HIGHEST priority,  select

the one that contains the fastest modem.

Intelligent Multi-port cards use a single shared IRQ. This is

the one you want to give highest priority to.

Zmodem          Technical Reference Manual          Page  22

Zmodem License

Please Read Carefully

WHEN you register Zmodem, you indicate your   acceptance   of

these terms.

1. License Grant. International TeleCommunications will grant

you  a non-exclusive license to use one copy of Zmodem  on  a

single  computer or on a single terminal connect to a  single

computer. Neither concurrent use on two or more computers nor

use  in a local area network is authorized  without  advanced

written  consent of International TeleCommunications and  the

payment of additional license fees.

2. International TeleCommunications' Rights. You  acknowledge

and   agree   that  Zmodem  is  a  proprietary   product   of

International   TeleCommunications,   protected   under   U.S

copyright  law,  international treaty provisions,  and  trade

secret laws of general applicability. You further acknowledge

and  agree  that all rights, title, and interest  in  and  to

Zmodem    are   and   shall   remain    with    International

TeleCommunications.

3. Other Restrictions. You may not lease or rent Zmodem.  You

may not disassemble, decompile, or reverse engineer Zmodem.

4.   Zmodem   is  licensed  "AS   IS,"   and    International

TeleCommunications disclaims any and all warranties,  whether

express  or  implied,  including  (without  limitation)   any

implied  warranties  of  merchantablity  or  fitness  for   a

particular purpose.

5. Limitation of Liability. International TeleCommunications'

cumulative  liability to you or any other party for any  loss

or  damage  resulting from any claims,  demands,  or  actions

arising out of or relating to this agreement shall not exceed

the license fee paid to International TeleCommunications  for

the   use  of  Zmodem.  In  no  event   shall   International

TeleCommunications  be liable for any  indirect,  incidental,

consequential, special, or exemplary damages or lost profits,

even if International TeleCommunications has been advised  of

the possibility of such damages. Some states do not allow the

limitation  or  exclusion  of  liability  for  incidental  or

consequential  damages, so the above limitation or  exclusion

may not apply to you.

6.  Governing Law. This license agreement shall be  construed

and  governed  in accordance with the laws of  the  State  of

Missouri.

Zmodem          Technical Reference Manual          Page  23

7. Severability. Shall any term of this license agreement  be

declared  void  or unenforceable by any  court  of  competent

jurisdiction,  such declaration shall have no effect  on  the

remaining terms hereof.