CfgCyrix Device Driver version 1.0

Tom Warren/Tech Guys SW

September, 1992

Updated January, 1994

CfgCyrix is intended to allow the technical user to upgrade their

386DX-based system to take advantage of the new, inexpensive i486-

like Cyrix Cx486DLC CPU.

While it is possible to get a performance increase just by installing

a 486DLC in place of the existing i386 CPU, it seems foolish not to

take advantage of the 486DLC's built-in 1K cache. I wrote CfgCyrix to

allow you to twiddle the cache config settings to achieve maximum

performance and maximum compatibility with your current software/hardware.

On a typical system (OPTi 386/33, with 64KB write-back cache), I

recorded the following results (Landmark Speedbar 2.00):

*base system, i386/33 CPU:    51.99 MHz

same system, Cx486DLC CPU:   78.32 MHz  (Cyrix cache disabled, default)

same system, Cx486DLC CPU:  108.29 MHz  (Cyrix cache enabled, /E1 /F1)

* 4MB RAM, 64KB WB cache enabled, no coprocessor, clean CONFIG.SYS

CfgCyrix command line syntax is available by 'typing' the file to

the console:

TYPE CFGCYRIX.SYS

CfgCyrix version 1.0  Copyright (C) 1992 Tom Warren/TechGuys SW

Usage: DEVICE=CFGCYRIX.SYS /A0 /B1 /C0 /D0 /E1 /F1 /H0 /K0 /R0 /S0

A0 = A20M# input pin (A20 mask) disabled, A1 = enabled

B0 = BARB (flush on HOLD) disabled, B1 = enabled

C0 = 2-way set-associative cache, C1 = direct-mapped cache

D0 = Debug flag disabled, D1 = enabled (verbose initialization info)

E0 = mark 640K-1Meg region as cacheable, E1 = mark region as noncacheable (NC0)

F0 = FLUSH# input pin (cache flush) disabled, F1 = flush enabled

H0 = set first 64K of each 1Meg boundary cacheable (real/V86 mode),

H1 = set first 64K of each 1Meg boundary noncacheable (NC1)

K0 = KEN# input pin (cache enable) disabled,K1 = cache enabled

R0 = RPLSET/RPLVAL# output pins disabled (floated), R1 = enabled

S0 = SUSP# input/SUSPA# output pins disabledS1 = enabled

Typically, the only bit that really needs to be set is F1, which

enables the FLUSH# pin.  It might also be prudent to set E1, which

disables caching of the 640K - 1Meg expansion ROM region. Also, I

now recommend that the BARB# bit be set (/B1), to flush the cache

whenever a DMA access occurs (floppy drives, sound boards, etc.).

Note that on the so-called ALPHA part (Rev A of the 486xLC) there

is a bug that disables caching from _512K_ to 1Meg when E1 is set.

I'm not sure how you can distinguish the ALPHA part from the BETA

(Rev B) part, but Cyrix may know if you call them @ (214) 234-8387.

They should have a data book available for the 486DLC, also.

Also, note that the Cyrix will appear as a 486 to most (all)

benchmark/diagnostic software, which is OK, except when it comes

to timing.  The Cyrix CPU executes some instructions much faster than

the i486, and may confuse code that is attempting to calculate

the CPU speed of the chip.  Use WhatCPU.COM, included in this

archive, if you need to check the true CPU clock speed.

I recommend that you put

DEVICE=\CFGCYRIX.SYS /B1 /E1 /F1

as the first line in CONFIG.SYS, but definitely before QEMM, 386MAX,

etc.  Note that in this case, CFGCYRIX.SYS is in the root directory.

Change the path to point to where you put the CFGCYRIX.SYS file.

Experiment with the settings until you achieve the best performance

on your system.  Run all of your "sensitive" software, such as

Windows, QEMM, etc., until you're happy with the settings, then

'lock' them into your CONFIG.SYS and enjoy!

Users of CfgCyrix and WhatCPU must accept this disclaimer of warranty:

The CfgCyrix and WhatCPU utilities are supplied as is.

The author disclaims all warranties, expressed or implied,

including, without limitation, the warranties of merchantability

and of fitness for any purpose.  The author assumes no liability

for damages, direct or consequential, which may result from the

use of CfgCyrix or WhatCPU.

CfgCyrix and WhatCPU are "shareware programs" and are provided at

no charge to the user for evaluation.  Feel free to share them with

your friends, but please do not give them away altered or as part of

another system. The essence of "user-supported" software is to provide

personal computer users with quality software without high prices, and

yet to provide incentive for programmers to continue to develop new

products.

If you find CfgCyrix and/or WhatCPU to be useful programs and continue

to use them after a reasonable trial period (30 days), I would appreciate

a contribution of $5.00 for my time and knowledge.  This will entitle you

to updates and bug fixes, as well as any new utilities I might distribute.

Anyone distributing CfgCyrix or WhatCPU for any kind of remuneration must

first contact me (via CompuServe) for authorization.

You are encouraged to pass a copy of CfgCyrix and WhatCPU along to

your friends and collegues for evaluation.  Please encourage them

to register their copy if they find that they can use it.

Tom Warren

4513 E. South Fork Drive

Phoenix, AZ  85044

(602) 940-3232

[CompuServe 76167,1572]