LPTMON version 1.0

By Celso Minnitti Jr. Public domain software. Copy freely.

LPTMON10.ZIP has these files:

�����������������������������������������������������ͻ

� LPTMON.ASM	8086 Assembly language source code    �

� LPTMON.COM	Executable program		      �

� LPTMON.DOC	This document			      �

� L.BAT 	Batch to compile and link LPTMON.ASM  �

�����������������������������������������������������ͼ

Introduction

������������

LPTMON is a Terminate and Stay Resident (TSR) program that will

display the values of the ports 378h (data port), 379h (status port),

3FAh (control port), and how many times an IRQ7 has been occured so the

user can monitor a printer at LPT1. I am including the full source code

of LPTMON so you can change it as you like.

Port 378h printer data output

�����������������������������

�7�6�5�4�3�2�1�0�

� � � � � � � ����� data bit 0, hardware pin 2

� � � � � � ������ data bit 1, hardware pin 3

� � � � � ������� data bit 2, hardware pin 4

� � � � �������� data bit 3, hardware pin 5

� � � ��������� data bit 4, hardware pin 6

� � ���������� data bit 5, hardware pin 7

� ����������� data bit 6, hardware pin 8

������������ data bit 7, hardware pin 9

Port 379h printer status register

���������������������������������

�7�6�5�4�3�2�1�0�

� � � � � � � ����� 1 = time-out

� � � � � �������� unused

� � � � ��������� 1 = fault,	pin 15

� � � ���������� 1 = on-line,  pin 13

� � ����������� 1 = paper end (out of paper),  pin 12

� ������������ 0 = Acknowledge,  pin 10

������������� 0 = busy,  pin 11

Port 37Ah printer control register

����������������������������������

�7�6�5�4�3�2�1�0�

� � � � � � � ����� 1 = output data to printer,  pin 1

� � � � � � ������ 1 = auto line feed,  pin 14

� � � � � ������� 0 = initialize printer,  pin 16

� � � � �������� 1 = select input,  pin 17

� � � ��������� 0 = IRQ disable,1=IRQ enable for ACK

�������������� unused

Signal Name	   25-Pin Connector    Description

�����������	   ����������������    ����������������������������������������

Strobe                    1            Output signal indicating the eight

data lines can be read

Data bit 0-7		 2-9	       Five volts = logical one

Zero volts = logical zero

Acknowledge		 10	       Input signal indicating data received

Busy			 11	       Input signal indicating do NOT transmit

Paper end		 12	       Input signal indicating printer is

out of paper

On-line 		 13	       Input signal indicating printer is

on-line, ready to receive

Auto line feed		 14	       Output signal selecting the line-feed

option. If the signal is low, the

printer is commanded to automatically

feed one line when it detects a carriage

return; If the signal is high, an

explicit line-feed character is required

Fault			 15	       Input Status indicating an error

Initialize printer	 16	       Output signal resetting the printer to

its default operating parameters

Select input		 17	       Output signal to switch the printer on

and off line

Ground			 18-25	       Ground

How to contact the Author

�������������������������

If you have any comments or suggestions about LPTMON you can reach

me at 617-235-4018 or write to:

Celso Minnitti Jr

139 Linden St

Wellesley, MA 02181