The data log file is started with a 6 byte header.  The data stream is

read and logged to the log file as follows:

The first byte is always F0.

Bytes 2 & 3 are the TIME in LSB MSB order.

The value of these bytes is the half the number of

seconds since midnite.

Example:

The SAMPLE.LOG (dated 05/21/93) file begins with the bytes:

F0 DC 50 95 2E 05

DC 50 is the time the file begins.  The decimal value of

50DC is 20700.  20700 / 1800 = 11.5  which indicates the

file was begun at 11:30 a.m.

Value of MSBLSB / 1800 = Hours since midnite

Bytes 4 & 5 are the DATE.

Byte 4 indicates the month and day while byte 5 indicates

the year.  January begins with 01, February with 21,

March with 41, and so on.  The first day of the month

is added so that the 12th day of January would be 0D.

The 31st day of January would be 1F.  (Don't forget, the

month begins at 00 which is January 0, not January 1.)

Byte 5 indicates the year.  1994 is 30 until the 9th

month when 31 is used. 08/01/94 is E1 30 and 09/01/94 is

01 31.   1995 is 32 & 33, etc.

To convert date to binary representation:

Yr% = (Year - 1950) + (Year - 1990)

If Month% > 8 then Month% = 1 : Yr% = Yr% +1

Mo% = (Month% * 32) - 32 + day%

LSB = CHR$(Mo%)

MSB = CHR$(Yr%)

To convert binary representation to date:

da% = VAL("&H" + RIGHT$(LSB$,1))

IF Mo% MOD 2 = 1 THEN Day% = Day% + 16

Yr% = VAL("&H" + MSB$)

Yr% = ((Yr% + 140) / 2) + 1900

Byte 6 is apparently always 05, presumably because the entries

are made at 5 minute intervals.

The data from the Ultimeter II weather station is logged

efficiently in log files by ULTIITSR because values are only

recorded if they have not changed since the previous entry.  For

example, if the Wind Direction and Temperature are the same as

the last time they were logged, only the Wind Speed is logged.

The AUTOWX.EXE program, as of version 3.1 uses this same format.

In the SAMPLE.LOG the entries are:

02 00 02 00 04 00 7B 00

These bytes indicate "upper" rainfall amount:  02 00

The next four bytes are the "lower" rainfall:  02 00

The next four bytes are:        04  Wind direction

00  Wind speed

7B  Temperature  (LSB)

00  Temperature  (MSB)

After the header is written, data is logged in the more efficient format

showing only changes since the last reading.  For example, if only the

temperature changed, the entry would be 10 97 00 where 10 is the

indicator that only a temperature value follows.  97 00 is the

temperature (+56) in LSB MSB order.  Rain amount is represented by 08H,

Wind Direction by 40H, Wind Speed by 20H and Temperature

by 10H.  All other combinations are the result of adding these values.

ie.  Temp change & Wind Speed Change:  10H + 20H = 30H  The values would

then follow 30H;  Wind Speed, 1 byte and Temp 2 bytes (LSB MSB).  Note

that the rain amount and temperature are the only values that use 2

bytes.

The original ULTIITSR program uses a value of 80H to indicate when rain

amounts are reset to 0.  AutoWx does not use the same algorithm but

writes a complete new header when rain amounts are reset.  This saves

several lines of code but adds only a couple of bytes to the log file

when the reset is performed.

The temperature, as documented by Peet Bros., is the data value

minus 56.  This allows for readings as low as -56F.

I have also noticed that the viewer (Ver 1) incorrectly displays the

wind speed.  I don't know if this is due to a bug or to some conversion

that may be going on in the code but, in any case, if you look at the

values recorded in the log file and compare them to those displayed by

the viewer, they don't match.  You can also compare the values displayed

on the U-II and you'll see that they do not match the values recorded in

the log file.  The numbers are also fouled up if you choose other than 5

minute intervals when listing tables.

By the way, I use MS-DOS Debug to view and work on log files when

experimenting/snooping around.  If you are not familiar with

DEBUG, be careful.  You can mess up an entire disk.

To view a log file, type in DEBUG SAMPLE.LOG at the DOS prompt.

Then, at the minus sign (-), type D and press RETURN.  Repeat to

see more.  Enter "Q" at the - sign to quit and return to DOS.

Those familiar with Debug, or those that have other utility

programs, such as a BIN to HEX program can change bytes, etc.  If you

are not familiar with DEBUG don't try doing anything but (D)isplaying

and (Q)uitting files.  Two log files can be appended by using:

COPY /B file1+file2 BIGFILE.LOG

If you have comments or if you develop other U-II software,

please contact me by mail or by packet at N5RKN.#WTX  BBS

73 de Les, N5KOA

Mon.       1994            00 - No changes                 0000 0000   0

JAN     01 30              08 - Rain                       0000 1000   8

FEB     21 30              10 - Temp                       0001 0000  16

MAR     41 30              18 - Temp & Rain                0001 1000  24

APR     61 30              20 - Speed                      0010 0000  32

MAY     81 30              28 - Speed & Rain               0010 1000  40

JUN     A1 30              30 - Temp & Speed               0011 0000  48

JUL     C1 30              38 - Temp, Speed & Rain         0011 1000  56

AUG     E1 30              40 - Direction                  0100 0000  64

48 - Dir & Rain                 0100 1000  72

SEP     01 31              50 - Dir & Temp                 0101 0000  80

OCT     21 31              58 - Dir, Temp & Rain           0101 1000  88

NOV     41 31              60 - Direction & Speed          0110 0000  96

DEC     61 31              68 - Dir, Speed, Rain           0110 1000  104

70 - Dir, Speed, & Temp         0111 0000  112

78 - Dir, Speed, Temp & Rain    0111 1000  120

A0 - Time, Date, Rain Reset                128+

E0 - Time, Date, Rain Reset                128+

N = 10             E = 04          S = 08          W = 0C

NNE = 01           ESE = 05        SSW = 09        WNW = 0D

NE = 02            SE = 06         SW = 0A         NW = 0E

ENE = 03           SSE = 07        WSW = 0B        NNW = 0F