PowerGlove Interface for Parallel Port on IBM

BIG NOTE:

Assemble at YOUR OWN RISK.  I have assembled this

cable successfully from these directions, BUT any

damage resulting from construction or use of this

cable is YOUR problem, not mine.

(just in case... <g>)

The Connector:

* 1

7 * * 2

6 * * 3     Looking AT the connector on glove cable

5 * * 4

Pinout:

1: Ground

2: Clock

3: Latch

4: Data

5: N/C (lightgun)

6: N/C (lightgun)

7: +5V

Connecting it to Parallel Port:

Glove      Printer Port

-----      ------------

1           18  GND

2            2  D0

3            3  D1

4           13  SLCT (input)

Power:

Connect pin 7 on glove to any location with +5V from

the computer.  Pin 1 on any gameport is a good place.

Or, use the keyboard connector.  The +5V on keyboard

should be pin 5 (may be wrong here, I'm using an XT

pinout) as follows:

Looking at back of computer:

3   1

5   4

2

The Cable:

I highly recommend buying one of the extender cables (the

Curtis Super Extendo cable is around $7 for 2 cables) to

chop up for the connector.  This makes it easy to also use

other Nintendo devices and allows TESTING the glove and

still being able to exchange it if defective.  (Plus you get

a long cable to the control box).

CODING

------

I'm including the latest code developed for the glove.  The

code is from the glove-list-mailing.  The glove-list-mailing

is an information exchange set up on internet to work on the

problem of solving problems in interfacing the glove and

discovering the hires mode.  The code is commented and uses

a glitch-reduction method for VERY clean sampling.  You will

need to adjust the N & D defines to match the timing on your

machine until you get a steady sample.

The code is currently set for LPT2. (INPORT = 0x279, OUTPORT

=0x278).  For LPT1, change INPORT to 0x379 and OUTPORT to

0x378.

Sorry, but this code is for Turbo C/C++/Borland C++.  It uses

the BGI library for the graphics cursor.  It is not difficult

to adapt it to Microsoft C (or other), though.

All the important info for reading is in the comments.  (BTW,

this is so new, I just received it today; MUCH better than

my original testing code <g>)

Chris Babcock

72657,2126

10/18/91

******EXTRA*********

Thanks to Chris for providing the code, which is courtesy of Dave Stampe

(whose internet address is in the code comments, so I won't repeat it here)

As Chris mentioned above, it is written with/for Borland compilers, and as

such, uses the Borland BGI library functions. I have TurboC++, and will try

to get around to changing the code to make it a free-standing executable, but

there is not enough time in my day, so if somebody wants to do this, feel free

to make the changes, rather than waiting for me! There are timing parameters

to tweak as well. Perhaps someone would like to do some work on this and

upload it to library13 of COMART?

John Eagan

76130,2225

Section leader,Virtual Reality section of Compuserve Computer Art forum

*********************