Documentation for Extra Entity REMoval version 1.0         August 7, 1994

Jon Fleming CIS 70334,2443

WHAT IS EEREM?

EEREM is a package written in C for the AutoCAD ADS environment.

It removes a large set of possible "extra entities" from an AutoCAD

drawing.  An "extra entity" is one which adds no information to the

drawing, such as duplicate lines.  Extra entities can arise from many

situations.  One situation that can give rise to MANY extra entities

is projecting a 3D wireframe model into 2D with PROJECT.LSP (included

with AutoCAD) or the equivalent.

Extra entities are inconvenient because they increase drawing file

size, lead to unexpected OSNAP results, and cause clogging of pens in

pen plotters forced to trace the same path several times.

BOILERPLATE

EEREM is public domain.  You may do what you want with it; please

credit me if you use the code or any significant portion of it in

your programs.

I expect EEREM to do what it is described as doing.  However,

EEREM is supplied "as-is" with no warranty of mercantability or

fitness for a particular purpose.  I accept no responsibility for its

operation or damages of any kind.

If you bring problems to my attention, I will do what I can to

resolve them.

WHAT DO I NEED TO USE EEREM?

This incarnation of EEREM has been tested under AutoCAD Release

12 c3 DOS and AutoCAD Release 12 c4 Windows. It may run under higher

release numbers, I cannot determine that at this time.  If

appropriately compiled, it _should_ run on other platforms.

For some reason I don't understand, EEREM does not work as well

on polylines under Windows as it does under DOS.

EEREM will NOT work under AutoCAD Release 11 or previous.

WARNING:  Although I cannot prove it, I think that earlier

releases of AutoCAD 12 had bugs in the ads_ssget() "X" interface which

led to EEREM missing many entities it should delete.  If EEREM does

not do the job for you, and you are running an older release of R12,

upgrade to the current release.

HOW DO I USE EEREM?

First, copy EEREM.EXP and EEREM.DCL into a directory (or

directories) that is (are) listed in the AutoCAD "ACAD" environment

variable.

EEREM must be loaded into memory.  At the AutoCAD "Command:"

prompt, you may type:

(xload "eerem")

(note:  once you have loaded EEREM, you need not do it again until you

have exited and re-entered AutoCAD.  This is different from LISP

programs which you must load every time you move from one drawing to

another without exiting AutoCAD).

Once EEREM is loaded, at the AutoCAD "Command:" prompt, type:

ddeerem

Then click on the "Select Entities for Checking" button, select a set

of entities to check, and click on the OK button.

See "ADVANCED EEREM USAGE", below, for hints and suggestions on

how to use EEREM efficiently; explanations of the other items in the

dialog box; and using EEREM from your own AutoLISP or C program.

WHAT ENTITIES DOES EEREM REMOVE?

First, a pair of definitions:

duplicate entity = an entity that covers EXACTLY the same extents as

another entity.

underlying entity = an entity that DOES NOT cover exactly the same

extents as another entity but is completely

covered by that other entity.

For example, if there's a line from (0,0,0) to (1,0,0), then another

line from (0,0,0) to (1,0,0) is a DUPLICATE entity.  Lines from (for

example) (0,0,0) to (0,0,0) or from (0.25,0,0) to (0.75,0,0) are

UNDERLYING entities.

Briefly (see "DETAILED ENTITY REMOVAL SPECIFICATIONS", below, for

more detail), EEREM removes:

All line duplicates and lines that underlie other lines (including

"zero-length" lines if they underlie a non-zero-length line).

All circle duplicates and arcs that underlie circles (circles

underlying circles are duplicate circles!).

All arc duplicates and arcs that underlie arcs.

All duplicate points (points underlying points are the same as

duplicate points!)

Most 2D and 3D (but not mesh) polyline duplicates and most 2D and 3D

(but not mesh) polylines that underlie other 2D and 3D (but not

mesh) polylines

WHAT COMMON EXTRA ENTITIES ARE _NOT_ REMOVED?

Different-type entities, such as polylines underlying lines.

(such entities are often created by PROJECT.LSP; all versions that

I've examined project all circles and arcs as polylines even when a

line or arc or circle would be more appropriate.  I haven't examined

the Release 12 PROJECT.LSP).

Entities that appear to overlay each other on the screen but are

actually far enough apart for EEREM to consider them different.

Entities that overlap, but neither entity completely covers the

other.

HINTS

EEREM will run _much_ more slowly if there are a lot of entities

in the drawing that are not involved in EEREM's operation.  This is

because EEREM asks AutoCAD to search the entire drawing database for

entities that may be deleted, and does so many times.  (Other

algorithms that I thought of proved to be much slower in 99% of the

cases).

Always run EEREM in a drawing that contains as few entities as

possible.  The AutoCAD WBLOCK command is often useful for creating

temporary drawings on which to run EEREM.

ADVANCED EEREM USAGE:  EASY LOADING

AutoCAD Release 12 includes a sophisticated automatic loading

facility for command-line-callable functions.  To activate this, if

you DO NOT have an S::STARTUP function in ACAD.LSP, add the following

to ACAD.LSP:

(defun S::STARTUP ()

(autoxload "EEREM" '("DDEEREM"))

)

If you already have a S::STARTUP function in ACAD.LSP, add the

following to it:

(autoxload "EEREM" '("DDEEREM"))

For usage from AutoLISP, the following works:

(if eerem

(eerem sel_set)

(if (xload "EEREM")

(eerem sel_set)

(prompt "\nCan't locate EEREM.EXP!")

)

)

ADVANCED EEREM USAGE:  FROM THE "COMMAND:" PROMPT

I hope the three check boxes at the top of the dialog box are

self-explanatory.  For example, say there's a line from (0,0,0) to

(1,0,0) on layer "0", and another line from (0,0,0) to (1,0,0) on

layer "DUMMY".  If the "Entities on different layers are different"

check box IS NOT checked, EEREM will remove one of these entites

(which one depends on their order in the set of entities selected for

checking).  If the "Entities on different layers are different" check

box IS checked, EEREM will not remove either entity.  The other two

check boxes behave in an analogous fashion.

There is one aspect that may be counterintuitive.  The colors or

linetypes BYLAYER or BYBLOCK are always considered different from an

explicit color or linetype.  For example, say there are two lines that

are identical in every respect, including what layer they are on,

except one has color BYLAYER and one has color YELLOW.  Say also that

the default color of the layer is YELLOW.  The two lines appear

identical on screen, but if the "Entities with different colors are

different" checkbox is checked EEREM will _not_ remove either line.

You probably don't want to change the "Tolerance" value, but you

can try it if you want.  Briefly, the tolerance is a measure of how

far apart points must be before EEREM considers them unequal (see

"DETAILED ENTITY REMOVAL SPECIFICATIONS", below, for more

explanation).  The tolerance is in drawing units.

ADVANCED EEREM USAGE:  FROM AUTOLISP

The dialog box version of EEREM may be called from AutoLISP by:

(ddeerem)

There is no return value.

To call the EEREM core engine without putting up a dialog box,

use:

(eerem sel_set checklayer checkltype checkcolor tolerance)

where:

sel_set is a selection set of entities to check, and is required.

checklayer is NIL to count entities on different layers as the same,

non-NIL to count entities on different layers as different.  checklayer

is optional (default NIL) unless one or more of the following arguments

are supplied.

checkltype is NIL to count entities with different linetypes as the

same, non-NIL to count entities with different linetypes as different.

checkltype is optional (default NIL) unless one or more of the

following arguments are supplied.

checkcolor is NIL to count entities with different colors as the same,

non-NIL to count entities with different colors as different.

checkcolor is optional (default NIL) unless one or more of the

following arguments are supplied.

tolerance is a real number that measures how close points must be to

be considered the same (well, approximately; see "DETAILED ENTITY

REMOVAL SPECIFICATIONS", below, for more detail).  tolerance is

optional.  The default is 0.00001.

(eerem ...) returns NIL if an error was detected or T otherwise.

ADVANCED EEREM USAGE:  FROM ADS

Call ddeerem through ads_invoke().  The return values are RSRSLT

(no error detected) or RSERR (can't find DCL file).

Call eerem through ads_invoke() with a list corresponding to the

AutoLISP parameters.  The return values are RSRSLT (corresponding to

the AutOLISP "T" return values, above) or RSERR (corresponding to the

AutoLISP "NIL" return values, above).

I haven't tested this.

DETAILED ENTITY REMOVAL SPECIFICATIONS:  GENERAL

All entity removal is based on a "current" entity and a

"candidate" entity.  The current entity is one of the set of entities

that was selected for extra entity removal, and must be a LINE,

non-mesh POLYLINE, CIRCLE, ARC, or POINT.  Other entity types, in the

set of entities selected for extra entity removal, are skipped.  The

candidate is the same type of entity as the current entity but MAY OR

MAY NOT be in the set of entities selected for extra entity removal;

all possible candidates in the drawing are checked.  The candidate is

the entity which will be deleted if it meets the criteria listed in

the following sections.

All combinations of current and candidate entities are checked.

Most of the filtering of candidate entities is done through the

ads_ssget() "X" mechanism, yielding a (usually) small set of

candidate entities that require further checking.  I think it is a

failure of the ads_ssget "X" mechanism to return the proper set of

entities that makes EEREM work improperly in early versions of R12

when the viewpoint is not "plan UCS".

Note that entity thickness IS NEVER CHECKED; if two entities are

the same except for thickness, one of them will be removed!

The following sections do not discuss whether or not the current

and candidate entity's layer, color, or linetype matter.  If the

appropriate flag(s) is(are) set, through the dialog box or through a

parameter to the function, unsuitable candidates are filtered out by

ads_ssget() "X".

DETAILED ENTITY REMOVAL SPECIFICATIONS:  LINES

The candidate line is deleted if both its endpoints lie within a

cylinder of radius "tolerance", centered on the current line and the

same length as the current line.

Lines shorter than 0.0001 times "tolerance" are not considered as

"current" entities, because the algorithm blows up for coincident

current line endpoints.  The presumption is that such short (or even

zero-length) lines are extremely likely to be underlying some other

line.

DETAILED ENTITY REMOVAL SPECIFICATIONS: CIRCLES & UNDERLYING ARCS

When the current entity is a circle, a candidate circle or arc is

deleted if:

its center lies within a cube of side two times "tolerance", centered

on the center of the current circle

AND

the candidate's radius is within plus or minus "tolerance" of the

current circle's radius

AND

the distance from the tip of the candidate's extrusion vector to

EITHER the tip of the current circle's extrusion vector OR the tip of

a unit vector directly opposite the current circle's extrusion vector

is less than "tolerance".

DETAILED ENTITY REMOVAL SPECIFICATIONS:  POINTS

A candidate point is deleted if it lies within a cube of side two

times "tolerance", centered on the current point entity.

DETAILED ENTITY REMOVAL SPECIFICATIONS:  ARCS

When the current entity is an arc, a candidate arc is deleted if:

its center lies within a cube of side two times "tolerance", centered

on the center of the current arc

AND

the candidate's radius is within plus or minus "tolerance" of the

current arc's radius

AND

the start and end points of the candidate arc are on or between the

start and end points of the current arc

AND

the distance from the tip of the candidate's extrusion vector to

EITHER the tip of the current arc's extrusion vector OR the tip of

a unit vector directly opposite the current arc's extrusion vector

is less than "tolerance".

DETAILED ENTITY REMOVAL SPECIFICATIONS:  POLYLINES

Both the current and candidate polylines may not be 3D polygon

meshes; such entities are skipped if they occur in the set of

entities to check.

Ready?  This is the BIG ONE!  Onward ...

A candidate polyline is deleted if:

its elevation is within plus or minus "tolerance" of the elevation of

the current polyline

AND

the candidate's start and end widths are within plus or minus

"tolerance" of the current polyline's start and end widths,

respectively

AND

the candidate's polyline type and curve fit type are the same as the

current polyline

AND

the distance from the tip of the candidate's extrusion vector to the

tip of the current polyline's extrusion vector is less than

"tolerance" (note that, unlike circles and arcs, polylines with

extrusion vectors exactly opposite to that of the current polyline

are not checked for deletion)

AND

for each successive vertex of the candidate, there is a vertex of the

current polyline that:

immediately follows the last current polyline vertex that matched

the previous candidate polyline vertex

AND

matches the current candidate polyline vertex.

In the above, "matches" means that:

the distance from the candidate vertex to the current polyline vertex

is less than "tolerance"

AND

the candidate vertex's start and end widths are within plus or minus

"tolerance" of the current polyline vertex's start and end widths,

respectively

AND

the candidate vertex's bulge factor is within plus or minus

"tolerance" of the current polyline vertex's bulge factor

AND

the distance from the tip of the candidate vertex's tangent vector to

the tip of the current polyline vertex's tangent vector is less than

"tolerance"

AND

the current vertex's flag bits (group 70) are the same as the flag

bits of the current polyline vertex.

(Whoof!  That was almost as hard to write down precisely as it was to

figure out!)

COMPILING EEREM:

EEREM was compiled for DOS or Windows using Watcom C/C++-32 9.5.

A .MAK file is included (in the source code package) for Watcom WMAKE.

Hints:  be careful when asking the compiler to do agressive

optimization; when I used "opmaxet" EEREM appeared to work but never

removed any polylines.  Be sure to delete *>OBJ or TOUCH *.C when

changing from compiling for Windows to compiling for DOS or vice

versa.