ELECTRICAL DESIGN SOFTWARE

Energy Cost Version 2.5  Documentation

(c) Copyright 1993-1994 Electrical Design Software

Licensed Material. All Rights Reserved.

ELECTRICAL DESIGN SOFTWARE

12 Vassar Drive Pelham, NH 03076

Phone or Fax: (603) 893-9008

TABLE OF CONTENTS                                     PAGE

-----------------                                     ----

Introduction ......................................... 1

Definition of shareware . ............................ 1

Disclaimer Agreement ................................. 2

What is Energy Cost 2.5 .............................. 3

System Requirements .................................. 4

Files ................................................ 4

Command Line Options ................................. 4

Ordering Forms or Computer Paper for Reporting ....... 5

Installation ......................................... 5

Getting Started ...................................... 5

Energy Cost Tutorial.................................. 6-15

Help System .......................................... 16

General Overview of the Main Menu Selections.......... 17-19

Each Data Entry Field Explained....................... 19-23

The Electric Bill Information Screen.................. 24-27

Data Base Subjects ................................... 28-29

Reporting - Print or File ............................ 29

Special Keys ......................................... 30

Quick Access Keys from the Main Screen ............... 30

Command Buttons Main Work and Names Screen ........... 31

Command Buttons Calculate Kilowatt Hour Screen ....... 31

Pop up Calculator..................................... 32

Lighting Schedules ................................... 33-35

Motor Schedules ...................................... 36

Power Factor Schedules ............................... 37

Formulas ............................................. 38

Setting Screen Colors................................. 39

Support............................................... 39

I N T R O D U C T I O N

-----------------------

Electrical Design Software is a New Hampshire based company that

was established in 1992.  Our organization's primary objective is

to produce quality software for the electrical industry at a

reasonable price.

ENERGY COST 2.5 requires no special computer knowledge, and is

designed to accommodate both the amateur and professional user.

Context Sensitive Help Screens have been programmed into this

application in order for you, the user, to continue working quickly

and efficiently.

Most importantly, here at Electrical Design Software, YOUR comments

are considered essential, for the development of future software

products.  With your input we will design programs to accommodate

the Electrical Industry and other specialized fields.

DEFINITION OF SHAREWARE

~~~~~~~~~~~~~~~~~~~~~~~

Shareware distribution gives users a chance to try software

before buying it.  If you try a Shareware program and continue

using it, you are required to register. Individual programs differ

on details -- some request registration while others require it,

some specify a maximum trial period.  With registration, you get

anything from the simple right to continue using the software to

an updated program.

Copyright laws apply to both shareware and commercial

software, and the copyright holder retains all rights, with a

few specific exceptions as stated below. Shareware authors are

accomplished programmers, just like commercial authors, and the

programs are of comparable quality. (In both cases, there are

good programs and bad ones!) The main difference is in the method

of distribution. The author specifically grants the right to copy

and distribute the software, either to all or to a specific group.

For example, some authors require written permission before a

commercial disk vendor may copy their Shareware.

Shareware is a distribution method, not a type of software.

You should find software that suits your needs and pocketbook,

whether it's Commercial or Shareware. The Shareware system makes

fitting your needs easier, because you can try it before you buy.

And because the overhead is low, prices are low also. Shareware

has the ultimate money-back guarantee -- if you don't use the

product, you don't pay for it.

Page 1

DISCLAIMER - AGREEMENT

~~~~~~~~~~~~~~~~~~~~~~

Users of ENERGY COST 2.5 must accept this disclaimer of

warranty: ENERGY COST 2.5 is supplied as is.  Electrical Design

Software disclaims all warranties, expressed or implied, including,

without limitation, the warranties of merchantability and of

fitness for any purpose. The author assumes no liability for

damages, direct or consequential, which may result from the use of

ENERGY COST 2.5.

ENERGY COST 2.5 is a "Shareware Program" and is provided at

no charge to the user for EVALUATION.  Feel free to share it with

your friends, but please do not give it away altered or as part of

another system.  The essence of "user-supported" software is to

provide personal computer users with quality software without high

prices, and yet to provide incentive for programmers to continue to

develop new products.

If you use ENERGY COST 2.5 after a 30 day trial period, you

MUST make a registration of $64.00 to Electrical Design Software.

The *$64.00 registration fee will license one copy for use on any

ONE computer at any one time.

You must treat this software just like a book. An example is

that this software may be used by any number of people and may be

freely moved from one computer location to another, so long as

there is no possibility of it being used at one location while it's

being used at another. Just as a book cannot be read by two

different persons at the same time.

Commercial users of ENERGY COST 2.5 must register and pay for

their copies of ENERGY COST 2.5 within 30 days of first use or

their license is withdrawn.  Site or distribution license

arrangements may be made by contacting Electrical Design Software.

Anyone distributing ENERGY COST 2.5 for any kind of

remuneration must first contact Electrical Design Software at the

address listed for authorization. This authorization will be

automatically granted to distributors recognized by the (ASP) as

adhering to its guidelines for shareware distributors, and such

distributors may begin offering ENERGY COST 2.5 immediately

(However Electrical Design Software must still be advised so that

the distributor can be kept up-to-date with the latest version of

ENERGY COST 2.5).

You are encouraged to pass a copy of ENERGY COST 2.5 along to

your friends for evaluation.  Please encourage them to register

their copy if they find that they can use it.  All registered users

will receive a copy of the latest version of ENERGY COST

Page 2

WHAT IS ENERGY COST - VERSION 2.5

---------------------------------

Energy Cost 2.5 is an information producing tool that will

calculate the cost of any electrical load or system. Accurate and

instant information is a KEY aspect when making financial decisions

on your electrical system.

With Energy Cost 2.5 you can analyze the SAVINGS when changing to

energy efficient equipment.  Compare the cost of one lighting

system to another.  Project how much a NEW piece of equipment will

cost to operate.  See the savings on using power during off-peak

times.  Study the most efficient mode of operation.  Identify power

inefficiencies and much more.

Energy Cost 2.5 has a complete data base system to store all load

data and electric bill information. Context and hyper-text help

systems and energy usage tables for lighting and motor loads are

built into the program.

Energy Cost 2.5 will produce TOTAL or COMPARISON reports, in cost

kilowatt hours or demand charges.

Page 3

SYSTEM REQUIREMENTS

-------------------

ENERGY COST 2.5 requires the following hardware and/or software

to operate:

1.  600K free memory - 286 or higher CPU.

2.  IBM(R) PC/XT/AT or true "compatible" running MS-DOS or PC-DOS

Operating System version 3.1 or higher.

3.  One high-density floppy-disk drive or a hard disk drive (hard

disk recommended)

4.  Expanded memory suggested, but not required.

FILES

-----

You have the following files for the ENERGY COST 2.5 program:

1.  ENERGY.EXE         Main program.

2.  ENERGY.MDB         Data file will be created on first start.

3.  ENERGY.HLP         Help screens.

4.  ENERGY.DOC         Documentation.

5.  ENERGY.INI         Initialization file.

6.  FEEDBACK.DOC       Program feedback.

7.  ISAMPACK.EXE       Utility to pack data file.   (in reg ver)

8.  ISAMREPR.EXE       Utility to repair data file. (in reg ver)

9.  VBDRT10E.EXE       Run time module.

COMMAND LINE OPTIONS

--------------------

When starting ENERGY you can set the monitor to 43 or, 50

line mode from the command line with the following command.

For 43 line mode type  ENERGY 43  then press ENTER.

For 50 line mode type  ENERGY 50  then press ENTER.

On 43 line mode the main work screen will be at the top leaving a

lower section to pull down the other screens. On 50 line mode the

main work screen will be at the top and the Names screen will setup

at the bottom. The main work screen is stationary while all other

screens are moveable and some are re-sizeable.

Moving windows

--------------

With the keyboard use Ctrl + F7 to activate the window move

feature. Use the arrow keys to place the window at the desired

location. Press ENTER to set the window in the new location. The

cursor must be on the window you want to move when using the

keyboard method.  With the mouse click the top bar and drag to the

desired location.

Page 4

ORDERING FORMS OR COMPUTER PAPER FOR REPORTING

----------------------------------------------

New England Business Service, Inc offers compatible forms for all

Electrical Design Software programs.  The advantage of forms is to

enhance the appearance of the printed reports.

NEBS Compatible Forms:

Computer paper ..........9911

Computer paper ..........9901

Computer paper .........12406

Multipurpose Form .......9331

For Fastest Ordering Call Toll Free  1-800-388-3810

WHEN ORDERING you must use this mail code number 11028

INSTALLING ENERGY COST 2.5

--------------------------

To install ENERGY, preform the following steps:

1. Start the computer as usual.

2. Insert the disk in   drive A: or B:.

3. From that drive type INSTALL & press ENTER.

Example on drive A:

Insert the disk into drive A:

Type  A:  then press ENTER.

Type  INSTALL  then press ENTER.

The install utility takes a moment to load.  Follow the

instructions.  Install will check your configuration, available

disk space, etc.

GETTING STARTED

---------------

o Go to the directory that Energy is in.

o From that directory type  ENERGY  then press ENTER.

Example:  From the Energy directory on drive C:

The command line may look like this C:\ENERGY>

Type  ENERGY  then press ENTER.

Page 5

ENERGY COST TUTORIAL

--------------------

Welcome to Energy Cost - a professional software program that is

especially designed for engineers, energy consultants, electrical

contractors, electrical power companies and students. Gain the

competitive edge with advanced PC technology and individual

know-how. This tutorial is designed to familiarize you with the

major features that Energy Cost has been widely known for.

You will feel at ease with these step-by-step instructions.  So

let's begin!

1.  First, start Energy Cost by going to the directory containing

the Energy Cost files. This may be the default directory

ENERGY  or any other directory that you may have specified

during installation. From that directory type ENERGY, then

press ENTER.  This will load the program and present the main

work screen.

2.  Remember to display a help message about the task you are

performing, press F1.  To exit help press ESC.

3.  We will continue by entering EXISTING LIGHTING in the tag

field.  The tag field is used to characterize the information

on the main work screen. As we gather various records the tag

field is one of five indexes we can use to find a specific

record.  Pressing ENTER will advance to the next field.

We must now enter the cost per kilowatt hour at peak time.  We

may enter the rate directly or we can calculate the rate using

the electric bill information screen.

4. To calculate the rates utilizing the electric bill screen,

access the electric bill information screen by selecting

(calculate kilowatt hour cost)  from the menu under utility.

Or for quick access from the main  work screen press the Ctrl

+ K keys.

5.  We will continue by entering the information on our electric

bill. This screen is not only a utility but this information

will be saved in the data file ENERGY.MDB along with the main

work screen and the names screen as one record. Let's go over

each field.

Page 6

6.  The meter number and service period are for two purposes.

1 - When printing a Kilowatt hour charge break-down report

2 - For future reference

Type in the sample information below

Example:

Meter #:  A12345678

Service period:  Nov 30 to Dec 30 1993    30 days

7.  Continue with the Peak Kilowatt hours. This is the total

kilowatt hours used during peak time. You can find this on

the electric bill under the heading of KWH Usage.

Example:

Meter readings            KWH

RATE        PRESENT  PREVIOUS         USAGE

----        -------  --------         -----

Peak         20523     19085          86280 <- Value

Off-Peak     16879     15604          76500

Enter the value 86280 into the Peak KWH:  Press ENTER.

8.  Now we want to enter the cost peak KWH usage $.  This is the

charge we pay for peak kilowatt hours.

Example:

Usage Charge: 86280  KWH  X  .03784  =  3264.84 <- Value

Enter the value 3264.84 into the cost peak KWH usage $:  Press

ENTER.

As soon as we press enter,  ENERGY will calculate the base

cost per peak kilowatt hour. Please note two things at this

point.

A. Some electric bills show the cost per kilowatt hour in

dollar format (.03784) = 3.784 cents. ENERGY shows the cost

per kilowatt hour in cents format (3.7840) = 3.784 cents.

This makes the figures more readable when analyzing the

rate structure.

B. ENERGY sets the value two additional decimal spaces to the

right.

Page 7

9.  Continue with the Off-Peak Kilowatt hours. This is the total

kilowatt hours used during off-peak time. You can find this

on the electric bill under the heading of KWH Usage.

Example:

Meter readings             KWH

RATE       PRESENT  PREVIOUS          USAGE

----       -------  --------          -----

Peak         20523    19085           86280

Off-Peak     16879    15604           76500 <- Value

Enter the value 76500 into the Off-Peak KWH:  Press ENTER.

10.  Enter the cost off-peak KWH usage $. This is the charge we pay

for off-peak kilowatt hours.

Example:

Usage Charge:   86280  KWH  X  .03784  =  3264.84

76500  KWH  X  .02346  =  1794.69 <- Value

Enter the value 1794.69 into the cost off-peak KWH usage $:

Press ENTER.

As soon as we press enter ENERGY will calculate the base cost

per off-peak kilowatt hour.

11. To get the value of our cost per kilowatt hour we must add

any other charges that may be on the electric bill. Let's start

with the fuel charge. What ENERGY will do is take the total

cost for the fuel charge and distribute it among the total peak

and off-peak kilowatt hours.

12. Enter the cost for fuel charge,  3838.35  then press ENTER.

We can now see that the cost per kilowatt hour has adjusted

upward.  The fuel charge of $3838.35 is divided by the total

kilowatt hours to give each kilowatt hour a portion of the fuel

charge cost.

Page 8

13. Again we may have additional charges, However this time we can

put in any heading such as:

* Type in the heading Oca charge $    Press ENTER.

* Type in the cost          188.82    Press ENTER.

The next three fields after fuel charge work exactly the same

as the fuel charge with the exception of being able to insert

your own heading.

14. Again we may have additional charges,

* Type in the heading Esc charge $    Press ENTER.

* Type in the cost             .33    Press ENTER.

The cost per kilowatt adjusted upward.

15. Type in  Rate settlement credit $  then press ENTER.

Enter the credit -133.48 then press ENTER.  Yes ENERGY will

take negative numbers and adjust the per kilowatt hour rate

accordingly.  We now have the cost per kilowatt hour based on

our actual electric bill.

16. Since we have a demand charge on the electric bill let's

continue.  Otherwise we would jump ahead to section 23. Sending

the rates to the main screen. If we do not have a demand charge

on the electric bill, leaving the Demand $ field on the main

screen empty will automatically take demand reporting out of

the reports.

17. Bring the cursor to the Per KVA cost $. In this area we can do

some very important calculating to verify the cost of

demand charges.  Most power companies charge the higher of 90%

of the actual KVA or 100% of the actual KW which ever is

higher.  We need to know the demand charge information to set

the demand adjustment factor when calculating the demand

charges.  Start by entering the cost per KVA as listed on our

electric bill.

Example:

Demand Charge:   461.1 KVA  X  8.490  =  3914.74

^

Value

Enter the value 8.49 into the Per KVA cost $:  Press ENTER.

Page 9

18. Enter the total cost for demand charges at Demand $.

Example:

Demand Charge:   461.1 KVA  X  8.490  =  3914.74 <-Value

Enter the value 3914.74 into the Demand $:  Press ENTER.

As soon as we press ENTER we can confirm our billed KVA to be

461.1 KVA. What ENERGY did was to divide $3914.74 by $8.49

19. Enter the Actual KVA

Example:          Actual

Demand

-----

512.4  KVA <- Value

417.0  KW

Enter the value 512.4 into the Actual KVA:  Press ENTER.

As soon as we press ENTER we can confirm that we are getting

billed for 90% of our actual demand. 461.1 billed KVA is 90% of

the 512.4 actual KVA.  The adjustment factor field now has a

value of 90.

20. The billed KVA is already entered into the billed KVA field.

If a change is made in the billed KVA field, the Demand $ field

and the Adjustment Factor will adjust accordingly. Press ENTER

to continue to Actual KW.

21. Enter the Actual KW

Example:          Actual

Demand

-----

512.4  KVA

417.0  KW  <- Value

Enter the value 417 into the Actual KW:  Press ENTER.

This value is used to:

* Set the adjustment factor when the actual KW is greater than

the calculated value for billable KVA.

* Calculate a power factor based on the actual KW divided by

the actual KVA times 100 = Service Power Factor.

Page 10

* Future reference.

This will print on the Kilowatt hour break-down report.

Example: 417KW  /  512.4KVA  X  100  =  81.38% Power Factor

22. The adjustment factor is already entered into the adjustment

factor field. This value is entered for you after entering the

billed and actual KVA fields. The adjustment factor field is

not accessible and cannot be changed directly.

The adjustment factor is used in calculating demand charges.

For example if we have a 10 KVA load and the cost per KVA is

$8.490  we would expect the charges for our 10 KVA load to be

$84.90. However since we are paying for only 90% of the actual

KVA load our true cost for the 10 KVA load is.

10 KVA  X  8.490 per KVA  X  .90 adjustment factor  =  $76.41

In this example our cost is based on 90% of our actual demand.

If the Actual KW is greater than 90% of the Actual KVA the

adjustment factor would adjust based on the Actual KW.

23. With all this information in place press the Send Rates command

button by pressing the Alt + S keys or click it with the mouse.

This will place the rates in their respective locations and we

can enter the loads.

24. At this point we have defined four cost rates.

1. Cost per peak kilowatt hour.

2. Cost per off-peak kilowatt hour.

3. Cost per KVA of demand.

4. The adjustment factor.

25. Continue by entering some loads. Our first study will be to

analyze the lighting in the office area of the All American

Flag Manufacturing Company. Bob the facilities engineer has

proposed to convert some existing lighting to a more energy

efficient system.  Bob must determine EXACTLY what the new

lighting will save each month before making any decisions.

Page 11

26. What we have in the office area is a total of 155 - 2X4 4 lamp

recessed fluorescent fixtures. Our first objective is to list

the loads and the times they are on. The office personal work

from  7:30 am to 4:30 pm Monday through Friday with the

lighting on a half hour before and a half hour after each work

day.

27. Enter the statistics for the existing lighting system as shown

below.

Load              Volt   Amp   Ph  P-F  KVA   P-H   O-H  P-D  Op-D

----              ----   ---   --  ---  ---   ---   ---  ---  ----

150 - 2x4 4 lamp   120  240.0   1  98  28.8   9.0   1.0   23   23

5 - 2x4 4 lamp   120    8.0   1  98   1.0  13.0  11.0   23   23

5 - 2x4 4 lamp   120    8.0   1  98   1.0        24.0         8

28. Line 1: Our first load is the 150 fixtures that are working

at 120 volts.  The total amps for the 150 fixtures is .8 amps

per ballast times 2 ballast per fixture times 150 fixtures =

240 amps. Since the amps are based on single phase we have

a 1 in the phase field.

The power factor of the existing ballast is 98% therefore put

98 in the PF% field. Most high power factor ballast operate

from 10 % to 2% of unity or a power factor of 90% to 98%.

We listed our load description for the 150 fixtures that are on

from 7:00 am to 5:00 pm Monday through Friday. Since the peak

time for this utility company is from 8 am to 9 pm Monday

through Friday. The 150 fixtures are on for 1 hour during

off-peak time and 9 hours at peak time. This occurs for 23 peak

days during this particular month.

29. Line 2: We also have 5 night light fixtures that never shut

off. These fixtures are on during PEAK AND OFF-PEAK DAYS.

First, consider usage on peak days. The 5 fixtures are on for

24 hours a day OR 13 hours per day at peak time and 11 hours

per day at off-peak time for 23 peak days.  Unlike the 150 on

line one that operate on peak days only.

We need to define the 5 fixtures two times once for peak days

(line 2) and once for off-peak days (line 3).  (Line 2 covers

Monday through Friday). If the load is used on days that are

entirely off-peak days we must list that usage on a new line.

Page 12

30. Line 3: The same 5 fixtures are also on for 24 hours a day

at off-peak time for 8 off-peak days.

(Line 3 covers Saturday & Sunday)

31. We have just established the criteria for the existing lighting

system. To complete this record we need to put a name on this

record by accessing the names screen.  Press the Alt + S keys

or click the NAME command button with the mouse.

32. Fill in the Prepared For and Prepared By information.

In the Header #1 field type the following.

Existing lighting 155 - 2x4  4 lamp  std ballast, std lamps.

33. Return to the main work screen by pressing the Alt + S

keys or click the screen command button with the mouse or the

easy way by pressing the Esc key once.

34. From here we will enter the proposed lighting into a new

record. We can do this two ways.  By selecting New Record and

entering the information for the proposed lighting system into

a new record.  Or copy the existing lighting record into a new

record and change the relevant fields.

35. Copy the existing lighting record by selecting copy this

record to a new record from the menu. When ever we leave a

record for any reason ENERGY will check if the record has been

changed in any way and verify that we want to save the changes.

When we copied the record we can see the amount of records

changed from one to two.

36. Select Yes.  Energy will update the existing lighting record

and copy this record to a new record.  We now have two copies

of the same record, we will modify the 2nd record to become our

proposed lighting.

Page 13

37. Start by entering Proposed lighting in the TAG field. Since

the Voltage, Phase, Power Factor & Times are the same we need

only change the KVA or Amps field. If we change the Amps field

energy will automatically update the KVA. If we change the KVA

field energy will automatically update the Amps field.

38. Change the load description from 4 lamp fixtures to 2 lamp

fixtures.  Our proposed fixture is a 2 lamp fluorescent fixture

with a reflective housing, T8 lamps and one electronic ballast.

The amp rating on the ballast is .52 amps.

150 fixtures X .52 amps = 78 amps.  Enter 78 in the Amps field.

39. The Amps for the 5 fixtures is .52 times 5 fixtures = 2.6 amps.

Enter 2.6 in the Amps field.

40. Enter 2.6 amps for line 3 also. We now have the information

for the existing and proposed lighting systems.

41. Go to the names screen and enter the following in Header #1.

Proposed lighting 155 - 2x4  2 lamp T8, electronic ballast.

42. Print a report for the existing & proposed lighting systems.

But first a few notes about going from record to record.

Records are displayed in an ascending order based on the

current index. The default index when ENERGY is started is the

TAG field.  We can go from record to record by:

1. Pressing F2 to display a list box.  Pressing the first letter

of the information in the tag field will jump to that record.

2. Select the Next command button to go to the next record.

3. Select the Prev command button to go to the previous record.

4. Select the Find command button to bring up a dialogue box.

Enter the search criteria. EXAMPLE: Type  Existing  then

press ENTER. This will go to the record with the word

Existing in the tag field or the next closest match.

5. When the cursor is on the TAG field press Page Up to go to

the previous record or Page Down to go to the next record.

Page 14

43. Go to the EXISTING LIGHTING record & print a COST TO OPERATE

MULTIPLE MONTHS report. Enter 12 in the months field. Select

the Print Report command button.

44. TO PRINT A REPORT

* Press ALT to activate the main menu.

* Select Reports, press ENTER.

* Highlight the  Cost to Operate with Totals, press ENTER.

Accept the default or select your specific printer port, press

ENTER.  Your printer is now printing a detailed report.

Do the same for the PROPOSED LIGHTING record.

45. As we examine the report we can see the total cost for the

existing and proposed lighting systems to be.

Existing  $656.52 per month X 12 months =  $7,878.24

Proposed -$213.49 per month X 12 months = -$2,561.88

=======

Savings  $443.03 per month X 12 months =  $5,316.36 per year

With this solid information at hand Bob can make a positive

decision for his company.

Congratulations!  You have just completed the ENERGY COST 2.5

tutorial.

Page 15

HELP SYSTEM

-----------

The help system is a context help system and hyper-text Help

system.  Some help screens have hyper-links that relate to the

subject in discussion.

F1  =  Pressing the F1 key on any input field will bring up help.

SELECTING A HELP LINK

---------------------

With the keyboard Press TAB or Shift-TAB to select a link.

Press ENTER to accept and go to that subject.

With a MOUSE .... Click on phrases outlined by two triangles.

SCROLLING THE HELP WINDOW

-------------------------

With KEYBOARD ... press the cursor navigation keys.

With a MOUSE .... click on the horizontal or vertical scroll bars.

CLOSING THE HELP WINDOW

-----------------------

With KEYBOARD ... Press Esc or Ctrl-F4.

With a MOUSE .... Double click on the windows control box icon,

top left.

Contents ........ Display a Table of Contents for the

applications HELP. Press C to select Contents.

Search .......... Opens a dialogue box that lists all available

HELP topics.  Select a topic from the list box

and press ENTER. Press S to select Search.

Back ............ Jumps to the last HELP topic viewed. Press B to

select Back.

History ......... Lists the last 20 HELP topics viewed. Jump to a

topic by selecting from the list box and

pressing ENTER. Press T to select History.

Copy ............ Copies the contents of the current HELP topic

into an edit area. Select the text you want

copied to the clipboard and choose copy.

Choosing copy without selecting any text in the

edit area copies the entire HELP topic to the

clipboard. Press P to select copy.

Page 16

GENERAL OVERVIEW OF THE MAIN MENU SELECTIONS

--------------------------------------------

REPORTS

-------

All reports can be sent to the printer or an ASCII file.

Cost to Operate ............      Cost to operate with line item totals

with Totals                       and group totals.

Cost to Operate ............      Cost to operate with line item totals

Comparison                        only.

Cost to Operate ...........       Cost to operate with line item group

Multiple Months                   and multiple month totals.

Kilowatt Hour Usage .......       Kilowatt hour usage with line item

with Totals                       totals and group totals.

Kilowatt Hour Usage .......       Kilowatt hour usage with line item

Comparison                        totals only.

Kilowatt Hour, Cost .......       Daily kilowatt hour usage and cost,

& Demand                          monthly kilowatt hour usage, cost and

demand charge. Each line item is

broken down into a detailed report.

Demand Charges ............       Demand charge summary, user can

select from the reports menu to

report on peak time loads only or

peak & off-peak loads by selecting

the "Calculating Demand on Peak Only"

menu item.

Print Field Form ..........       Print a form that can be used to

collect information in the field.

Energy will Calc ..........       When Energy is started the default is

Demand on Peak Only               to report the demand charges on peak

loads only.   Select this menu item

to toggle demand reporting for

off-peak also.

Order NEBS Forms...........       Information for ordering NEBS

Compatible Forms.

Exit.......................       Exit ENERGY.

Page 17

DATA BASE

---------

Search for Records.......  Display a search list box to

search for records.

Copy this Record.........  Copy the entire contents of the

current record to a new record.

Set Index to Tag.........  Index records on the Tag field.

First Record.............  Make the first record current.

Last Record..............  Make the last record current.

Create New Record........  Create a new record.

Update...................  Update the current record.

Delete...................  Delete the current record.

Previous.................  Move to the previous record in the

indexing sequence.

Next.....................  Move to the next record in the indexing

sequence.

Find.....................  Display an input box to type in the

record to find. The search will take

place on the field that is the current

index.

UTILITY

-------

Calculate Kilowatt.......  Display the electric bill information

Hour Cost                  screen.  Use this work screen to

calculate  peak, off-peak and demand

charge values.

Set Screen Colors.......   Display a screen to adjust the various

screen colors.

Page 18

HELP

----

Contents.................  Various help hyper-link topics.

Search...................  Sort the help subjects alphabetically.

Context Help.............  Display help for the field the

cursor is in.

Special Keys.............  Screen navigation and functions.

Using Help...............  Explains about the help system.

About....................  Information about this program.

EACH DATA ENTRY FIELD EXPLAINED

-------------------------------

TAG

---

Use the Tag field to identify information on the main screen.

The Tag field is the only indexing field for the main screen.

When the cursor is on the Tag field you can move to the

previous record by pressing Page Up or move to the next

record by pressing Page Down.

Information in the tag field will not print on reports. This

field is for find and search only.

The data file will store up to 25 characters for this field.

PEAK RATE

---------

The cost per peak kilowatt hour.  A peak kilowatt hour is the

use of 1000 watts for one hour during peak hours.

This field will accept a value up to 50 cents.

Perform the following to calculate the value for the peak rate:

1. select calculate kilowatt hour cost from the menu.

2. Enter the information from the electric bill

3. Press the send rates command button. This will place the

calculated rate into the main work screen.

Page 19

OFF-PEAK RATE

-------------

The cost per off-peak kilowatt hour. An off-peak kilowatt hour

is the use of 1000 watts for one hour during off-peak hours.

This field will accept a value up to 50 cents.

Perform the following to calculate the value for the off-peak rate:

1. select calculate kilowatt hour cost from the menu.

2. Enter the information from the electric bill.

3. Press the send rates command button. This will place the

calculated rate into the main work screen.

DEMAND CHARGE

-------------

The Demand Charge is the cost per KVA of demand. This charge is

generally for commercial & industrial users.

This field will accept a value up to 50 dollars.

Below is an example of how the demand is measured by some utility

companies.

The Peak Hours demand for each month under ordinary load conditions

shall be the greatest of the following:

A. The greatest fifteen-minute peak occurring during the

peak hours period within such a month as measured in

kilowatts (KW).

B. 90% of the greatest fifteen-minute peak occurring during

the peak hours period of such a month as measured in

kilovolt-amperes (KVA).

LOAD

----

Identify the load being analyzed.  Example: 25 hp air compressor

The data file will store up to 25 characters for this field.

Page 20

AMPS

----

Amperes of the load.

If you make an entry into the Amps field with the Volts &

Phase fields greater than 0, a value will automatically be

calculated & inserted into the KVA field.

This field will accept a value up to 999 amps.

VOLTS

-----

Enter the voltage.

This field will accept a value up to 15000 volts.

Standard AC Voltages

110 - 120 - 208 - 220 - 240 - 440 - 480 - 550 - 600

2,200 - 2,300 - 2,400 - 4,000 - 4,160 - 6,600 - 6,900

11,000 - 11,500 - 13,200 - 13,800

PHASE

-----

Phase of the load. This field will accept 1 or 3.

When at the Phase field, and a value is in the Amps & Volts fields,

a non entry in the Phase field will cause it to default to 3

POWER FACTOR

------------

Power Factor of the load as expressed in percent.

Power factor is the ratio of the true power or watts to the

apparent power or voltamperes. The power factor is expressed

as a percentage.

This field will accept a value from 1 to 100.

Page 21

KVA

---

KVA or apparent power of the load. When the Amps, Volts & Phase

fields are greater than 0 a change in the KVA field will

automatically calculate & adjust the Amps.

This field will accept a value up to 25000 KVA.

PEAK HOURS

----------

Enter the peak hours per day.

This field will accept a value up to 24 hours.

The peak and off-peak hours for one line item cannot be greater

than 24.  For example a value of 15 peak hours and 10 off-peak

hours on the same line is not valid.

Check with the power company's customer representative to confirm

the peak hours as applied to your electric bill.

The following is a typical schedule of peak hours.

Peak hours will be from 8:00 am TO 9:00 pm daily Monday through

Friday, excluding holidays.

The utility company reserves the right to change these peak and

off-peak hours, but in no case will the off-peak hours be less

than eleven hours a day.

OFF-PEAK HOURS

--------------

Enter the off-peak hours per day.

This field will accept a value up to 24 hours.

The peak and off-peak hours for one line item cannot be greater

than 24.  For example a value of 15 peak hours and 10 off-peak

hours on the same line is not valid.

Check with the power company's customer representative to confirm

the off-peak hours as applied for your electric bill.

The following is typical schedule of off-peak hours.

Off-peak hours will be from 9:00 pm TO 8:00 am daily Monday

through Friday, and all day on Saturdays, Sundays and holidays.

The utility company reserves the right to change these peak and

off-peak hours, but in no case will the off-peak hours be

less than eleven hours a day.

Page 22

PEAK DAYS

---------

Enter the number of days that the load is using power during

the peak hours period.

This field will accept a value up to 9,999 Days.

The following is typical schedule for peak days.

8:00 am TO 9:00 pm daily Monday through Friday,

excluding holidays.

OFF-PEAK DAYS

-------------

Enter the number of days that the Load is using power during

the off-peak hours period.

This field will accept a value up to 9999 days.

The following is typical schedule of off-peak days.

9:00 pm TO 8:00 am daily Monday through Friday, and all day on

Saturdays, Sundays and holidays.

CUSTOMER INFORMATION

--------------------

The Name, City, State & Zip fields are indexing fields. To change

the index to the prepared for name, Select Set Index to Name In the

top menu of the Names screen.

When the cursor is in the Prepared For name field you can move

to the previous record by pressing Page Up or move to the next

record by pressing Page Down.

PREPARED BY INFORMATION

-----------------------

This information will print on all reports under the prepared

by heading.  If the Prepared By name field is empty no prepared

by information or heading will print on the report.

HEADERS AND FOOTERS

-------------------

If information is entered into the Header field it will print at

the top of all reports. The header is an excellent location to list

a description or purpose of the report.

If information is entered into the Footer field it will print at

the bottom of all reports.

Page 23

THE ELECTRIC BILL INFORMATION SCREEN

------------------------------------

METER NUMBER

------------

Enter the meter number indicated on the electric bill. This is

useful for future reference and when printing a report.

SERVICE PERIOD

--------------

Enter the service period indicated on the electric bill. This is

useful for future reference and when printing a report.

PEAK KWH

---------

Enter the value that reflects usage during peak time.

PEAK COST

---------

Enter the usage charge on the electric bill for kilowatt hour usage

during peak time.

OFF-PEAK KWH

------------

Enter the value that reflects usage during off-peak time.

OFF-PEAK COST

-------------

Enter the usage charge on the electric bill for kilowatt hour

usage during off-peak time.

FUEL CHARGE

-----------

Enter the fuel charge on the electric bill.

This dollar amount is distributed among the total peak and

off-peak kilowatt hours to calculate the actual cost per

kilowatt hour.

This field will accept a value up to 9,999,999.00 dollars.

Page 24

ADDITIONAL CHARGE

-----------------

Use this field to enter the description of a charge or credit.

You can enter any heading for this field.  ENERGY will save

the information in the data base file and use it for the printed

reports.

Example:  Oca charge

Example:  Rate settlement credit

Example:  Sales tax

This dollar amount is distributed among the total peak and

off-peak kilowatt hours to calculate the actual cost per

kilowatt hour.

This field will accept a value up to 9,999,999.00 dollars.

PER KVA COST

------------

Enter the cost per KVA for the demand charge.

This field will accept a value up to 50 dollars.

When you enter the information from the electric bill the

per KVA cost, Demand $ & the Billed KVA must all reflect the

figures on the electric bill. This will assure ENERGY that the

rates that are sent to the main screen via the Send Rates

command button are correct.

To Verify your input ENERGY will:

* Take Demand$ / Per KVA cost to verify the Billed KVA.

* Take Per KVA cost X Billed KVA to verify the Demand$

DEMAND COST

-----------

Enter the Total Demand Cost. This is the total you pay for the

demand charge.

This field will accept a value up to 9,999,999.00 dollars.

When you enter the information from the electric bill the

Per KVA cost, Demand $ & the Billed KVA must all reflect the

figures on the electric bill. This will assure ENERGY that the

rates that are sent to the main screen via the Send Rates

command button are correct.

Page 25

To Verify your input ENERGY will:

* Take Demand$ / Per KVA cost to verify the Billed KVA.

* Take Demand$ / Billed KVA to verify the Per KVA cost.

BILLED KVA

----------

Enter the Total billed KVA.

This field will accept a value up to 25000 KVA.

When you enter the information from the electric bill the

Per KVA Cost, Demand $ & the Billed KVA must all reflect the

figures on the electric bill. This will assure ENERGY that the

rates that are sent to the main screen via the Send Rates

command button are correct.

To Verify your input ENERGY will:

* Take Per KVA cost X Billed KVA to verify the Demand$

* Take Demand$ / Billed KVA to verify the Per KVA cost

ACTUAL KVA

----------

Enter the actual KVA as shown on the electric bill.

This field will accept a value up to 25000 KVA.

This value is used to:

* Calculate the adjustment factor.

* Calculate the Power Factor based on the electric bill

information.

ACTUAL KW

---------

Enter the actual KW as shown on the electric bill.

This field will accept a value up to 25000 KW.

This value is used to:

Page 26

* Calculate the adjustment factor when the Actual KW is higher

than the calculated value of KVA.

* Calculate the Power Factor based on the electric bill

information.

ADJUSTMENT FACTOR

-----------------

The adjustment is automatically entered for you when you

enter the Actual KVA & the Billed KVA. This value is used

to adjust the actual cost burden for the Demand Charge.

The adjustment factor field is not accessible and cannot be

changed directly.

The reason for the adjustment factor is that most electric

utility companies will allow for a Power Factor of 90%.

Therefore if you have a 10 KVA Load and the power company is

billing for 90% of the total KVA then the billable KVA on the

10 KVA load is 90% of the 10 KVA or 9 KVA. The adjustment factor

will assure that the reports reflect the true cost that you

incurred on the electric bill.

Example: Assume a per KVA cost of $10.00. When we look at the

electric bill we see that the actual KVA is 100 but the billed

KVA is 90.

This would cause the electric bill to look like this:

90 KVA X $10.00 per KVA = a Demand Cost of $900.00

Now let's calculate the demand cost of a motor load with a KVA

demand of 10 KVA. If we took the 10 KVA X $10.00 per KVA the

demand cost would be $100.00 This would be $10.00 higher than

we were actually charged on the electric bill for this motor.

Since we are paying 90% of the actual KVA the formula for the

motor must be:

10 KVA  X  $10.00 per KVA  X  .9 adjustment factor = $90.00

This value is used to adjust the Demand Cost on the reports.

Page 27

DATA BASE SUBJECTS

------------------

ENERGY has 5 Indexing fields.

Tag .................. Main work screen

Prepared for Name .... Names screen

Prepared for State ... Names screen

Prepared for City .... Names screen

Prepared for Zip ..... Names screen

Records are sorted in an ascending order based on the current

index.  When ENERGY is first started the default index is the Tag

field of the main work screen. To change the current index select

the desired index from the Names screen menu.

Four of the five indexes also have multiple sub indexes.

Prepared for Name    City    State

Prepared for State   Name    City

Prepared for City    Name    State

Prepared for Zip     Name    City

Example: The name index will sort based on the Name field

first, then as sub-index it will sort on City and

lastly on State.

DATABASE PACKING UTILITY

------------------------

ISAMPACK purges records marked for deletion and recovers disk

space formerly used by deleted records in 32K blocks.

When a record is deleted it is removed from the indexing order.

However the information remains in the data file. Using ISAMPACK

reconstructs the data file with active records only.

Syntax     ISAMPACK  ENERGY.MDB

DATABASE REPAIR UTILITY

-----------------------

ISAMREPR recovers databases that become corrupted; it recreates

them by systematically evaluating every table and index and using

every piece of internally consistent information in the file.

Syntax     ISAMREPR  ENERGY.MDB

Page 28

MAXIMUM FIELD LENGTH

--------------------

The Maximum Field Lengths for various fields is the data that

will be saved in the data base record.

TAG ...................25     LOAD DESCRIPTION ......20

PREPARED FOR NAME .....25     PREPARED FOR ADDRESS ..25

PREPARED FOR CITY .....20     PREPARED FOR STATE ....20

PREPARED FOR ZIP ......10     PREPARED BY  NAME .....25

PREPARED BY  ADDRESS ..25     PREPARED BY  CITY .....20

PREPARED BY  STATE ....20     PREPARED BY  ZIP ......10

HEADER   ONE ..........72     HEADER   TWO ..........72

FOOTER ................72

REPORTING - PRINT OR FILE

-------------------------

All reports can be sent to

1.  Printer ports LPT1, LPT2 or LPT3

2.  An ASCII file via a user supplied name.

When sending reports to the printer you  have the option to

produce multiple copies from 1 - 10 with the copies field.

When sending a report to file ENERGY produces an ASCII file

that you can edit with any word processor. If the default

left and right margins of your word processor are less than

the width of the report it may be necessary to set the

margins to accept the width of the report before loading the

report file.

o With Replace ENERGY will first check if the file name you

supplied already exist. If so a message box will confirm

if you want to continue and overwrite the existing file

or cancel the operation.

o With Append if the file already exist ENERGY will add the

report to the end of the existing file otherwise a new file

will be created. A convenient way to use append is to

produce multiple reports all into the same file.

Energy will produce the following types of reports:

o Cost to Operate with Totals.

o Cost to Operate Comparison.

o Cost to Operate Multiple Months.

o Kilowatt Hour Usage with Totals.

o Kilowatt Hour Usage Comparison.

o Cost to Operate, Kilowatt Hour Usage & Demand Charges.

o Demand Charges.

Page 29

SPECIAL KEYS

------------

ENERGY COST 2.5 uses the following operating functions:

1.   ENTER .............     Accept input & advance to the next field.

2.   TAB ...............     Advance to the next field. When in a help

window pressing tab will jump from one

hyper-link to the next.

3.   SHIFT + TAB .......     Move to the previous field.

4.   ALT ...............     Activate the top Menu.

5.   ALT + HOT KEY .....     Select menu item or command button. Hot

keys are highlighted.

6.   ESC ...............     Move towards the main screen & out of the

program.

7.   NAVIGATION ........     Up or down arrows keys.

8.   PAGE UP ...........     When at the Prepared For Name, page up

will move to the previous record.

9.   PAGE DOWN .........     When at the Prepared For Name, page

down will move to the next record.

10.  MOUSE .............     Select any field or button.

11.  F1 ................     Context help.

12.  F2 ................     Search list box.

12.  Ctrl + F4 .........     Close a window.

13.  Ctrl + F6 .........     Activate next window.

14.  Ctrl + F7 .........     Move a window.

QUICK ACCESS KEYS FROM THE MAIN SCREEN

--------------------------------------

15.  Ctrl + X ....  Exit program

16.  Ctrl + T ....  Set index to tag.

17.  Ctrl + P ....  Move to previous record.

18.  Ctrl + N ....  Move to next record.

19.  Ctrl + F ....  Find record screen.

20.  Ctrl + K ....  Quick access to Calculate KWH Cost screen.

21.  Ctrl + C ....  Copy the contents of this record to

a new record.

22.  Ctrl + H ....  Quick access to the contents help window.

23.  Ctrl + S ....  Quick access to the search help window.

Page 30

COMMAND BUTTONS MAIN WORK & NAMES SCREEN

----------------------------------------

New Rec....... Create a new record.

Update........ Update the current record. When a record is changed

a message box will request if Energy can update the

record before you exit.

Delete........ Delete the current record.

Previous...... Move to the previous record in the indexing

sequence.

Next.......... Move to the next record in the indexing sequence.

Find.......... Find a specific record. The search will take place

on the field that is the current index.

Calc.......... Bring up a pop up calculator.

Name.......... Move to the Name screen.

Exit.......... Exit the program.

Esc........... The Esc button will move out of the program.

COMMAND BUTTONS CALCULATE KILOWATT HOUR COST

--------------------------------------------

Send Rates............ Calculate the information on the electric

billing information screen and send the

peak, off-peak and demand rates to the

main work screen.

Clear Fields.......... Clear the fields on the electric billing

information screen.

Calculator............ Display a pop up calculator.

Main Screen........... Exit the electric billing information

screen & access the main work screen.

Page 31

POP-UP CALCULATOR

-----------------

Bring up a pop-up calculator. With the mouse you can move the

calculator around by placing the mouse cursor on the top of the

calculator, press the left mouse button and drag to a new screen

location.

c  -  Clear

ce  -  Clear last entry

+  -  Addition

-  -  Subtract

*  -  Multiply

/  -  Divide

=  -  Equals

%  -  Percent

To remove the calculator press one of the functions listed below:

1.    Press ESC,

2.    Click the exit button on the menu bar,

3.    Click the mouse on the background screen.

Page 32

LIGHTING SCHEDULES

------------------

Below is a general list of the rated WATTAGE for various types of

fixtures. Most ballast today have high power factor ballast that

bring the power factor to within 10% to 2% of unity.

The figures below represent the wattage or true power consumed by

the various fixtures and not the volt-amps or apparent power.

Please consider this if you are using these figures and reporting

on the demand charges.

Example:  A 2 Lamp 4' Standard lamp and Standard ballast fixture

is rated at 94 watts, therefore, 94 watts / 120 volts = .783 amps.

However one manufacture has the ballast listed at .8 amps. The

va at .8 amps is 120 volts X .8 amps = 96 va

With this information we can list the fixture at .8 amps instead of

the  .783 but in the power factor column we can put 98 instead of

100. (94 watts / 96volt amps = .979% Power Factor  rounded to

98%). Using this method we would have a report that has a 2% better

accuracy on the demand charges.

STD   - Standard ballast or lamp

EE    - Energy efficient lamp

ELIG  - Electronic or hybrid ballast

EEMAG - Energy efficient magnetic ballast

FOUR-FOOT FLUORESCENT SYSTEMS

-----------------------------

Description    Rated Wattage        Description    Rated Wattage

-----------    -------------        -----------    -------------

F40T12

1L4'STD/STD         57              1L4'STD/EEMAG       50

2L4'STD/STD         94              2L4'STD/EEMAG       86

3L4'STD/STD        151              3L4'STD/EEMAG      136

4L4'STD/STD        188              4L4'STD/EEMAG      172

1L4'STD/ELIG        46              1L4'EE/STD          50

2L4'STD/ELIG        72              2L4'EE/STD          86

3L4'STD/ELIG       110              3L4'EE/STD         136

4L4'STD/ELIG       144              4L4'EE/STD         172

1L4'EE/EEMAG        40              1L4'EE/ELIG         38

2L4'EE/EEMAG        70              2L4'EE/ELIG         60

3L4'EE/EEMAG       110              3L4'EE/ELIG         90

4L4'EE/EEMAG       140              4L4'EE/ELIG        120

F32T8

1L4'T8/EEMAG        37              1L4'T8/ELIG         30

2L4'T8/EEMAG        70              2L4'T8/ELIG         60

3L4'T8/EEMAG       107              3L4'T8/ELIG         90

4L4'T8/EEMAG       140              4L4'T8/ELIG        120

Page 33

FOUR-FOOT FLUORESCENT SYSTEMS

-----------------------------

Description    Rated Wattage        Description    Rated Wattage

-----------    -------------        -----------    -------------

F48T12

1L4'F48T12/STD      60              1L4'F48T12EE/STD    50

2L4'F48T12/STD     102              2L4'F48T12EE/STD    82

3L4'F48T12/STD     162              3L4'F48T12EE/STD   132

4L4'F48T12/STD     204              4L4'F48T12EE/STD   164

F48T12HO

1L4'HO/STD/STD      85              1L4'HO/EE/STD       80

2L4'HO/STD/STD     145              2L4'HO/EE/STD      135

3L4'HO/STD/STD     230              3L4'HO/EE/STD      215

4L4'HO/STD/STD     290              4L4'HO/EE/STD      270

FIVE-FOOT FLUORESCENT SYSTEMS

-----------------------------

2L5'STD/STD        135

SIX-FOOT FLUORESCENT SYSTEMS

----------------------------

1L6'STD/STD         95              2L6'STD/STD        173

1L6'VHO/STD/STD    180              2L6'VHO/STD/STD    300

1L6'T8/EEMAG        53              2L6'HO/STD/STD     206

3L6'STD/STD        213

EIGHT-FOOT FLUORESCENT SYSTEMS

------------------------------

F96T12

1L8'STD/STD        100

2L8'STD/STD        173              2L8'STD/EEMAG      158

3L8'STD/STD        273

4L8'STD/STD        346              4L8'STD/EEMAG      316

1L8'STD/ELIG        70              1L8'EE/STD          83

2L8'STD/ELIG       134              2L8'EE/STD         138

3L8'EE/STD         221

4L8'STD/ELIG       268              4L8'EE/STD         276

1L8'EE/ELIG         60

2L8'EE/EEMAG       123              2L8'EE/ELIG        109

4L8'EE/EEMAG       246              4L8'EE/ELIG        218

Page 34

HIGH INTENSITY DISCHARGE SYSTEMS

--------------------------------

MERCURY VAPOR

-------------

Rated Wattage                  Rated Wattage

-------------                  -------------

40 W Mercury Vapor     50      50 W Mercury Vapor     75

75 W Mercury Vapor     95     100 W Mercury Vapor    120

175 W Mercury Vapor    205     250 W Mercury Vapor    290

400 W Mercury Vapor    455    1000 W Mercury Vapor   1075

METAL HALIDE

------------

50 W Metal Halide      85      70 W Metal Halide      95

100 W Metal Halide     130     150 W Metal Halide     195

175 W Metal Halide     205     250 W Metal Halide     300

400 W Metal Halide     455     750 W Metal Halide     825

1000 W Metal Halide    1080    1500 W Metal Halide    1620

HIGH PRESSURE SODIUM

--------------------

35 W - HPS             45      50 W - HPS             65

70 W - HPS             95     100 W - HPS            130

150 W - HPS            190     200 W - HPS            245

250 W - HPS            295     310 W - HPS            365

400 W - HPS            460    1000 W - HPS           1100

LOW PRESSURE SODIUM

-------------------

35 W - LPS             60      55 W - LPS             85

90 W - HPS            130     135 W - HPS            180

180 W - HPS            230

Page 35

MOTOR AMPERE SCHEDULES

Table 430-148 Full-Load Currents in Amperes

Single-Phase Alternating-Current Motors

_______________________________________________________

HP         115v         200v         208v         230v

-------------------------------------------------------

1/6        4.4          2.5          2.4          2.2

1/4        5.8          3.3          3.2          2.9

1/3        7.2          4.1          4.0          3.6

1/2        9.8          5.6          5.4          4.9

3/4       13.8          7.9          7.6          6.9

-------------------------------------------------------

1          16            9.2          8.8          8

1         20           11.5         11           10

2          24           13.8         13.2         12

3          34           19.6         18.7         17

-------------------------------------------------------

5          56           32.2         30.8         28

7         80           46           44           40

10         100           57.5         55           50

-------------------------------------------------------

Table 430-150 Full-Load Currents*

Three-Phase Alternating-Current Motors

-------------------------------------------------------

HP       200v      208v      230v      460v      575v

-------------------------------------------------------

1/2       2.3       2.2       2.0       1.0       0.8

3/4       3.2       3.1       2.8       1.4       1.1

1         4.1       4.0       3.6       1.8       1.4

-------------------------------------------------------

1        6.0       5.7       5.2       2.6       2.1

2         7.8       7.5       6.8       3.4       2.7

3        11.0      10.6       9.6       4.8       3.9

-------------------------------------------------------

5        17.5      16.7      15.2       7.6       6.1

7       25.3      24.2      22.0      11.0       9.0

10        32.2      30.8      28.0      14.0      11.0

-------------------------------------------------------

15        48.3      46.2      42.0      21.0      17.0

20        62.1      59.4      54.0      27.0      22.0

25        78.2      74.8      68.0      34.0      27.0

--------------------------------------------------------

30        92.0      88.0      80.0      40.0      32.0

40       119.6     114.4     104.0      52.0      41.0

50       149.5     143.0     130.0      65.0      52.0

-------------------------------------------------------

60       177.1     169.4     154.0      77.0      62.0

75       220.8     211.2     192.0      96.0      77.0

100       285.2     272.8     248.0     124.0      99.0

-------------------------------------------------------

125       358.8     343.2     312.0     156.0     125.0

150       414.0     396.0     360.0     180.0     144.0

200       552.0     528.0     480.0     240.0     192.0

Page 36

POWER FACTOR SCHEDULES

----------------------

Many of the newer energy efficient motors have a

power factor rating listed on the name plate. This

value would represent the power factor at rated load.

The most accurate method for determining power factor would

be to measure the specific circuit with a power factor meter.

Below is a table showing average power factors for various

3 phase 1800 RPM motors.

These values will vary from ONE manufacture to another as

well as for different RPM values. Lower RPM motors have

lower power factor ratings and higher RPM motors have higher

power factor ratings.

Average Power Factors for Standard Industrial Design

3 phase Squirrel-Cage Induction Motors.

---------------------------------------------

Hp     1/2 load   3/4 load   Full load

---------------------------------------------

1/2   |    45    |    56    |    65

3/4   |    48    |    58    |    65

1    |    57    |    69    |    76

1   |    64    |    76    |    81

2    |    68    |    79    |    84

3    |    70    |    80    |    84

5    |    76    |    83    |    86

7   |    77    |    84    |    87

10    |    77    |    86    |    88

15    |    81    |    85    |    87

20    |    82    |    86    |    87

25    |    82    |    86    |    87

30    |    83    |    86.5  |    87

40    |    84    |    87    |    88

50    |    84    |    87    |    88

60    |    84    |    87    |    88

75    |    84    |    87    |    88

100    |    84    |    88    |    88

125    |    84    |    88    |    89

150    |    84    |    88    |    89

200    |    85    |    89    |    90

250    |    84    |    89    |    90

300    |    84    |    89    |    90

--------------------------------------------

Typical Power Factors

---------------------

1 phase induction motors  1/20 to 1hp ....... 55 to 75

average at rated load ....... 68

1 phase induction motors 1hp  to 10hp ........75 to 86

average at rated load ........82

Incandescent lighting ........100

Electric resistance heaters ........100

Page 37

FORMULAS

--------

OHM'S LAW 1 PHASE

-----------------

Watts   =   E  x I    =   Volts  X  Amps

Watts   =   E x R    =   (Amps squared) X  Ohm's

---------------------------------------------------

Volts   =   I X R     =   Amps X  Ohm's

Volts   =   P / I     =   Watts   /  Amps

Volts   =   (P / R)  =   Square of (Watts / Ohm's)

---------------------------------------------------

Ohm's   =   E / I     =   Volts /  Amps

Ohm's   =   E    =   Watts /  (Amps squared)

---------------------------------------------------

OHM'S LAW FOR A 3 PHASE LOAD WITH A POWER FACTOR OF 100%

--------------------------------------------------------

Watts   =   E  x I x 1.732   =   Volts  X  Amps  X  1.732

---------------------------------------------------------

Volts   =   P / (I X 1.732)  =   Watts  / (Amps  X  1.732)

----------------------------------------------------------

Amps    =   P / (E X 1.732)  =   Watts  / (Volts X  1.732)

----------------------------------------------------------

OHM'S LAW FOR A 3 PHASE LOAD WITH ANY POWER FACTOR

--------------------------------------------------

Watts   =   Volts  X  Amps  X  1.732  X  POWER FACTOR

---------------------------------------------------------

Volts   =   Watts  / (Amps  X  1.732  X  POWER FACTOR)

---------------------------------------------------------

Amps    =   Watts  / (Volts X  1.732  X  POWER FACTOR)

---------------------------------------------------------

Power Factor = Watts  / (Amps  X  Volts  X  1.732)

---------------------------------------------------------

Page 38

SETTING SCREEN COLORS

---------------------

Select "Set Screen Colors" from the top menu bar.  Highlight the

screen element you wish to adjust and select the forecolor and

backcolor.  Pressing TAB will move the cursor from one control to

the next.

OK ......   Close control panel and accept changes.

Cancel ......   Close control panel and cancel changes since last

save.

Default ......   Close control panel and set all colors to the

original settings.

Three dimensional - Toggle three dimensional on and off.

Window shadow - Toggle window shadow on and off.

SUPPORT

-------

Support for ENERGY COST 2.5 is FREE.  Please use any method you

want. When you call for support have the following available:

1. Your name or company

2. Program name and version

BY PHONE

--------

Monday through Friday 9:00 am to 5:00 pm Eastern Standard time

phone (603) 893-9008.  If we are unavailable at the time of your

call, please leave your name, phone number, fax number, message &

the best time to call back. Explain the details for the support

call.  Thank you.

BY FAX

------

24 hours a day 7 days a week call 603-893-9008

BY MAIL

-------

Send to:    Electrical Design Software

12 Vassar Drive

Pelham, NH 03076

Attn: Customer Support

Page 39